| Structure Number: |  | Crossing Name: |  |
| --- | --- | --- | --- |
| Road Name: |  | Road Number: |  |
| SLK: |  | Local Authority: |  |
| BMA: |  | Latitude: |  |
| Inspected By: |  | Longitude: |  |
| Inspection Date: |  |  |  |

## Have structural issues been found that require further investigation? (Y/N)

| Inspection Item | Defect |  | Comments (Including location and extent) | Maint. Required |
| --- | --- | --- | --- | --- |
|  | Yes | No |  |  |
| 1. Road Surface Signs and Delineators: missing, damaged, obscured Road Surface and Footpaths: material defects, surfacing defects, settlement, depressions, joint transitions, kerbing, shoulders |  |  |  |  |
| 2. Guardrails/Barriers Accident damage, connections, alignment, material defects |  |  |  |  |
| 3. Road Drainage Scuppers, drains, gully traps, erosion |  |  |  |  |
| 4. Waterways, Vegetation and Debris Vegetation and debris in waterways and clearance envelope Embankment erosion, scour, damaged guide-banks |  |  |  |  |
| 5. Footpaths Drainage, even surface, surface condition, railing |  |  |  |  |
| 6. Expansion Joints and Deck Joints Loose/damaged fixings, damaged/missing seals, damage to deck/nosings, obstructions in gap, gaps closed, decks in contact/damaged |  |  |  |  |
| 7. Bearings Bearings displaced or damaged, seating, corrosion, seized |  |  |  |  |
| 8. Superstructure Material defects or damage to beams/stringers, fasteners, soffit, cross bracing or coatings Debris/dirt build-up, impact damage, excessive movement/vibration, dampness through deck, condition of air release holes |  |  |  |  |
| 9. Substructure Material defects to piles, footings, walls or capbeams Movement of abutment or wing walls Substructure protection (bridges over road/rail) |  |  |  |  |

Defect Descriptions

| Material | Defect Descriptions |
| --- | --- |
| Concrete | Cracking (hairline: ≤0.1mm) (fine: >0.1 & ≤0.3mm) (medium: >0.3mm & ≤0.7mm) (heavy: >0.7mm), spalling, corrosion of reinforcement, rust staining, efflorescence |
| Steel | Bending, buckling, cracking, distortion, corrosion, protective coating damage/deterioration, loose fasteners |
| Timber | Splitting, crushing, rot, failure, termites/termite nest |
| Masonry | Cracking, opening of joints, mortar loss, bulging |
| Bituminous Surfacing | Cracking, crazing, breaking up, heaving, shoving, rutting |
| Protective Coating | Cracked, weathered, peeling, flaking, oxidising |

General Comments

Ancillary Items (such as service attachments etc.)

This bridge has been inspected in accordance with the requirements of the Main Roads Western Australia Bridge Inspection Manual for Level 1 Inspections.

# Signature:  	 Position:  	                                        Date:

| Structure Number: |  | Crossing Name: |  |
| --- | --- | --- | --- |
| Road Name: |  | Road Number: |  |
| SLK: |  | Local Authority: |  |
| BMA: |  | Latitude: |  |
| Inspected By: |  | Longitude: |  |
| Inspection Date: |  |  |  |

Photo 1:

Photo 2:

Photo 3:

Photo 4:

Photo 5:

Photo 6:

Photo 7:

Photo 8:

Photo 9:

Photo 10: