Planning and Technical Services Directorate

Road and Traffic Engineering Branch

TRIM File No 12/8087         Document No.  67/08/44	          Issue Date:  1/3/2012

TRIM Doc No D12#434814

MAPPING  STANDARD

This document is owned and controlled by the Survey & Mapping Manager.

The Senior Mapping Surveyor is the delegated custodian.

All comments for revision should be submitted to the Senior Mapping Surveyor.

Authorisation

As Executive Director of the Planning and Technical Services Directorate of Main Roads, I authorise the issue and use of this standard.

..............………………........................................................

EXECUTIVE DIRECTOR PLANNING TECHNICAL SERVICES DIRECTORATE

Date: ___/ ___/ ___

No controlled copies are issued for this standard.

The latest copy of this standard is located on the Main Roads Internet site.

REVISION STATUS RECORD

| Page No | Latest Revision No | Latest Revision Date | Revision Description/Reference |
| --- | --- | --- | --- |
| ALL |  | 1/3/2012 | Complete document revision. |

CONTENTS

1.	PURPOSE	5

2.	SCOPE	5

3.	REFERENCES	5

4.	DEFINITIONS AND INTERPRETATIONS	5

5.	AERIAL PHOTOGRAPHY CAPTURE	7

5.1	Landgate Registration	7

5.2	Aerial photography prints	7

6.	Entry Onto Land	7

7.	LANDOWNER CONTACT	7

8.	ENVIRONMENTAL IMPACT	8

9.	Photogrammetric control TARGETS	8

9.1	Control Target Material	8

9.1.1	Painted Targets on Roads	8

9.2	Photogrammetric Control Configurations.	8

9.2.1	Photogrammetric Ground Control for Class A.	9

9.2.2	GPS Photogrammetric Ground Control for Class B and C.	9

9.2.3	Block Configuration	9

10.	AEROtriangulation	10

11.	MAPPING	10

11.1	Mapping Format	10

11.2	Coordinate System and Datum	10

11.3	Data Capture	10

11.4	Mapping Accuracy	11

11.4.1	Direct comparison of audit points and mapping points on hard and soft surfaces.	11

11.4.2	Audit points and mapping interpolation points	11

11.5	Out of Tolerance areas	11

11.6	Terrain Definition	12

11.6.1	Digital Elevation Models	12

11.6.2	Contours	12

11.7	Detail strings	13

11.7.1	Road capture	13

11.7.2	Road markings	13

11.7.3	Bridges	13

11.7.4	Watercourses and High Water Marks	13

11.7.5	Vegetation	14

11.7.6	Circular and Square Manholes for Class A Mapping	14

11.7.7	Circular and Square Manholes for Class B & C Mapping	14

11.7.8	Power Lines	15

11.7.9	Power Poles / Telegraph poles	15

11.7.10	Light poles	15

11.7.11	High Tension Power Towers	15

11.7.12	Side Entry Pits for Class A Mapping	15

11.8	Mapping Codes	16

11.8.1	Main Roads mapping code table.	16

11.8.2	MX code compression.	16

11.8.3	2D and 3D codes.	16

11.8.4	“Special 3D” codes.	16

11.8.5	Linear Feature Codes	16

11.8.5.1	Centrally Located Linear Features	17

11.8.5.2	Electric Light Poles	17

11.8.5.3	Pavement Arrows	18

11.8.5.4	Left and Right Handed Features	18

11.9	Text Data	19

11.10	Mapping Extent String	19

11.11	Features not required to be captured.	19

11.12	Verification Survey.	20

11.12.1	Verification String Survey – Class A.	21

11.12.2	Verification Cross Survey – Class B and C.	21

11.12.3	Statistical analysis and verification statement.	22

11.13	Verification Survey String Data File	22

12.	FINAL data supply.	23

12.1	Mapping and Verification Report.	23

12.2	Naming MX Genio files.	23

12.3	Mapping Base Material.	23

13.	MERGING MAPPING AND DIGITAL GROUND SURVEY DATA.	24

14.	SUPPLY MEDIUMS FOR DIGITAL DATA.	24

APPENDIX A – Main Roads feature codes.	25

APPENDIX B – Mapping report example	32

# PURPOSE

The purpose of this standard is to establish Main Roads requirements for mapping work that is captured through photogrammetric mapping techniques.

This document addresses aerial photography, photogrammetric control and mapping capture requirements.  These topics can be used in isolation or in conjunction with each other to produce products to meet Main Roads accuracies and existing operating system requirements.

# SCOPE

This standard must apply to all work related to Mapping.

Advice and further information on this standard can be obtained by contacting the Senior Mapping Surveyor, Road and Traffic Engineering Branch, Main Roads.

# REFERENCES

The following documents relate to this procedure:

“Land Administration Act”

# DEFINITIONS AND INTERPRETATIONS

The following terms used in this procedure have the specific meanings indicated:

Mapping – spatial survey data captured from photogrammetric techniques.

Photogrammetry – mapping created from photographic images.

Contractor - the party named in the contract to perform the Services

Main Roads - Main Roads Western Australia.

MRWA - Main Roads Western Australia

Survey Portal – Software found online within the Main Roads Western Australia web site that contains information on coordinate systems and road reference marks.

UTM - Universal Transverse Mercator coordinate system.

Landgate – the Statutory Authority which maintains Western Australia’s official register of land ownership and survey information and is responsible for valuing the State's land and property for government interest.

GPS - Global Positioning System.

GNSS - Global Navigation Satellite System.

Cross Sectional Ground control – Control targets that are placed in pairs either side of the flight line - approximately opposite.

Strip Mapping – A single run of over lapping aerial photographs from which mapping is derived.

RTK GPS – Real Time Kinematic methodology used Global Positioning Systems

GDA94 – Geocentric Datum of Australia 1994

MGA94 - Map Grid Australia 1994

Local Project zone - a local transverse mercator projected plane coordinate system created by Main Roads to meet tolerances for building roads and bridges on the earths curved surface.

AHD - Australian Height Datum.

Hard Surfaces - An area where the upper soil profile is no longer exposed to the actual surface of the earth.  Hard surfaces include, but are not limited to, the formation of the road i.e. road shoulder, road surface, kerbs, sidewalks, ramps, driveways, buildings, bridges concrete structures and other features.

Soft Surfaces - An area where the upper soil profile is exposed or can be said to be natural surface.  Soft surfaces include sand, banks, fields, paddocks, rock outcrops, levees etc.

TIN – Triangulated Irregular Network.  Three dimensional points that are joined to the nearest surrounding points by vectors to create planes and thus define a surface.

3D - Three Dimensional (Easting, Northing and height)

2D - Two Dimensional (Easting and Northing)

MX – Road design software formerly known as MOSS software

SLK – Straight Line Kilometres.  The location system defined by Main Roads to define features along a road.

Left and Right hand rule –Defines what side of a feature a symbol is plotted.

Thalweg – From a longitudinal perspective it is the deepest point of a water feature that defines a river/stream/creek.  The lowest point of a stream in any given cross section

Soffit level – The height of the underside of a bridge.

Project Manager – Is the Main Roads individual that is responsible for the overall road project.

Contract Manager – Is the Main Roads individual that has a responsibility to provide a specific service through a contract for the project manager.

# AERIAL PHOTOGRAPHY CAPTURE

All new aerial photography captured must cover the areas outlined in the Main Roads requesting technical brief and if applicable must ensure that all mapping requirements are met.

All new aerial photography requested by Main Roads must have:

- 60% forward overlap and if applicable, a minimum of 30% side overlap.

- The requested road alignment located within the central 60% of the photographic frame.

- Be in colour.

## Landgate Registration

All photography must be registered at Landgate, being film or digital, with all rights assigned for public access.  It is the contractor’s responsibility to request Landgate’s 6 digit project numbers and to comply with Landgate’s requirements for film registration.

## Aerial photography prints

Aerial photographic contact prints and digital images are not required to be supplied to Main Roads unless specifically requested.

# Entry Onto Land

It is the consultant’s responsibility to seek and obtain permission from landowners, occupiers or management authorities before entering any property to undertake survey work.  Property includes Crown land which may consist of Reserves, National Parks or State Forests.  Survey work may include, but is not limited to, investigation, placement, coordination and removal of survey control targets and the collection of field audit points.

If access onto land is refused by the owners, the consultant will be required to immediately contact the Main Roads contract manager to discuss the need for entry onto the land.  If entry is required for the completion of the contract and there are no other alternatives, then Main Roads will arrange formal notification using delegated powers under the Land Administration Act.  A formal notice of entry requires Main Roads to provide 7 days notice to the owners.  The process to arrange the formal notice of entry may take some time and the consultant must liaise with the Project Manager to ensure disruption to the contract schedule is minimised.

# LANDOWNER CONTACT

The consultant shall maintain a written record of all contact made with landowners while working on the mapping contract.  This includes, and is not limited to, anything written via letters and emails, contact made verbally via phone calls or door knocking on properties.

Brief written records of all contact must be supplied as part of the mapping report.  Please see Appendix B Mapping Report for details to be recorded.

If further queries are required to be made by land owners with respect to the mapping project, then all questions are to be referred the Main Roads project manager.

# ENVIRONMENTAL IMPACT

All work must minimise any environmental impact.  Any breach of environmental and heritage legislation during the execution of works is the sole responsibility of the consultant.

The consultant shall ensure any disturbances are kept to an absolute minimum, and shall reinstate, clean-up and leave the site as close to its pre-disturbed condition on completion of any work or investigation.  All mapping target material must be removed before the completion of the contract.

New tracks shall not be formed, existing tracks altered, fencing cut, clearing carried out, or damage or disturbance made of any kind unless approved by the Main Roads project manager.

The Consultant shall be responsible for the cost of reinstating any damage to property resulting from their work.

# Photogrammetric control TARGETS

The planning, placement, size and coordination of photogrammetric control targets are to be determined by the contractor.  Sufficient control must be used to ensure that the required accuracies for the requested mapping are obtained.  The minimum requirement for photogrammetric control is defined in table 9.2.

The location of targets on the ground must minimise the entry and use of private property.  The placement of targets on private property must minimise the disturbance to the property.  For example, targets should be placed at the edge of tracks or sides of paddocks - not within cropped areas.

## Control Target Material

Due care must be taken to ensure that materials used for targets will not damage the environment or cause any harm to humans, animals or machinery.  Plastic targets must not be used in livestock areas.

All mapping control target material and fastenings (i.e. pins, wire, nails etc) must be removed from the field as soon as possible after the aerial photography has been captured to minimise the impact on property owners.  However, preplanned control points (but not target material) may be retained in urban areas if they are suitable for future works.  These remaining control points must be pre-approved by the Senior Geodetic Surveyor within Main Roads.

### Painted Targets on Roads

Painted targets are not to be used on any roads controlled by Main Roads.

Painted targets may be used on local or private roads where permission has been granted from the appropriate authority or owners.

## Photogrammetric Control Configurations.

The following are Main Roads minimum requirements for full cross sectional control configurations for single strip mapping photography.

Cross sectional control must be established on all bends and no further than the number of models listed in the following table.

| MAPPING CLASS | HORIZONTAL | VERTICAL |
| --- | --- | --- |
| A | 4 | 2 |
| B | 5 | 3 |
| C | 6 | 3 |

For example, Class A requires cross sectional horizontal targeting on every forth model and cross sectional vertical targets on every second model.

### Photogrammetric Ground Control for Class A.

Class A photography must have cross section ground control placed in accordance with the requirements of the table in 9.2.

Class A photogrammetric control must not use GPS coordinated exposures.

Horizontal control targets must be coordinated by using a network of distances to adjacent horizontal control marks.  The network must have redundant observations and be adjusted by the least squares method.  Horizontal and vertical control points must not be coordinated by radiation.

Horizontal and vertical control points can not be established by RTK GPS.

Vertical values for control points must be spirit levelled between at least two known and verified bench marks.  The difference between two known bench marks in a single run shall not exceed:

Difference = 12 √K, where

The maximum allowable difference is expressed in millimetres and K is the distance levelled in kilometres.

Spirit levelling misclose is to be distributed proportional to distance.  The final heights of the photo control points is to be the adjusted level.

Any errors or movement detected in existing road reference marks or Landgate survey control (benchmarks or standard survey marks) are to be reported to the Senior Geodetic Surveyor at Main Roads.

### GPS Photogrammetric Ground Control for Class B and C.

GPS exposures can be captured for Class B and C mapping as an alternative to the ground control configurations stipulated in table in 9.2.  When GPS exposures are used, 3D cross section ground control must be located on all bends and be no more than nine models apart.  Additionally, a minimum of one 3D control point must be established mid way between consecutive cross section pairs when there are more than 4 models between cross sectional control.

### Block Configuration

Control configurations for block photography must ensure that Main Roads classes for mapping are met.

# AEROtriangulation

A Least Squares bundle adjustment will be used on all strip aerial photography and a Least Squares block adjustment is to be used on a block of photography.  The resulting control outputs are to be of a standard to ensure the required mapping accuracies are met.

# MAPPING

Full detailed mapping must be captured and presented in accordance with this standard.

Main Roads requires the true shape of the natural surface of the earth and all man made features to be captured within the project area to enable it to improve existing roads and to plan and or design new roads.

To meet Main Roads requirements for mapping the consultant must comply with the following;

## Mapping Format

All mapping data must be supplied in MX Genio file format which must be compatible with Main Roads MXRoads design software.

## Coordinate System and Datum

Mapping data shall be lodged in the local Main Roads coordinate system.  This will be defined within the mapping brief.

If a mapping project runs through a number of adjoining zones, a minimum of 500 meters overlap is required between project zones.   The mapping data for each project zone must be supplied as seperate MX genio files.

Parameters for all project zones are available from the Senior Geodetic Surveyor and or through the “Survey Portal”, within the Main Roads web site.

## Data Capture

All mapping data captured will consist of point or string features.  Except for the features described in “11.8.5 Linear Features Codes”, all point features are defined by their centre and all string features are defined by the edge of the feature.  In some cases, Left and Right hand rule string code labels are available.

## Mapping Accuracy

Captured mapping must comply with the following accuracy table and the following 11.4.1 and 11.4.2 point comparison statements.

|  | HARD SURFACES |  | SOFT SURFACES |  |
| --- | --- | --- | --- | --- |
| MAPPING CLASS | VERTICAL | HORIZONTAL | VERTICAL | HORIZONTAL |
| A | ± 0.07 m | ± 0.10 m | ± 0. 10 m | ± 0.20 m |
| B | ± 0.20 m | ± 0.25 m | ± 0.30 m | ± 0.50 m |
| C | ± 0.30 m | ± 0.70 m | ± 0.50 m | ± 1.25 m |

When vertical comparisons are being made, any field audit point that falls into the horizontal tolerance of a mapped point is deemed to be the same point, unless it is proven otherwise.

### Direct comparison of audit points and mapping points on hard and soft surfaces.

95% of all audit points directly compared to mapping captured points must be within the tolerances of the requested class listed in table 11.4.

### Audit points and mapping interpolation points

On hard surfaces, 95% of all audit points compared to interpolated mapping points on a triangulated model should be within the tolerances of the requested class listed in table 11.4.

And

On soft surfaces, 95% of all audit points compared to interpolated mapping points on a triangulated model should be within twice the tolerances of the requested class listed in table 11.4.

## Out of Tolerance areas

Where large areas on the ground cannot be observed due to vegetation (i.e. thick tree foliage or lakeside reeds), an out of tolerance boundary string (Main Roads code XX**) may be observed.

All out of tolerance areas must have text contained within it and must provide an estimation of the accuracy of the area.  For example “Area out of tolerance -/+0.5m”.

The out of tolerance string must consist of ground points that meet the ground accuracy of the requested mapping.  The out of tolerance string must also be a closed string unless it passes out of the area required to be mapped.

Every effort must be made to observe ground points within the out of tolerance area even though they will exceed the maximum distance between mapped 3D points (see section 11.6) for the requested accuracy class.

The Senior Mapping Surveyor must be immediately notified when the out of tolerance string is used within a mapping project.

The mapping report must clearly state that there is an out of tolerance area.  In addition the metadata table must contain a note clearly highlighting this issue.

## Terrain Definition

Points and break lines are to be used to accurately define the shape of the earth’s surface.  Points and strings must define changes of grade to enable the mapping accuracies in section “11.4 Mapping Accuracies” to be met.  The maximum distance between 3D points on the same string or distance between terrain defining points shall not exceed the following values.

| MAPPING CLASS | POINT SPACING |
| --- | --- |
| A B C | 20 m 60 m 100 m |

### Digital Elevation Models

If a Digital Elevation Model (DEM) is requested in the technical brief, then the maximum horizontal distance between spot heights (Main Roads code “PRL*”) shall be as shown in the following table.

| MAPPING CLASS | SPOT HEIGHT POINT INTERVAL |
| --- | --- |
| A B C | 8 m 20 m 40 m |

Spot heights (Main Roads code PRL*) must not be captured or supplied on road surfaces.

### Contours

Contours must be supplied.  Contours must define the natural terrain and be smooth continuous strings.  Contours may be directly observed or generated in flat or undulated areas.  The methodology of contour production must be clearly stated within the survey report.

All new mapping contours must butt join existing Main Roads project contours if an existing mapped area is extended.

All contours must be presented as a “special purpose” three dimensional format when supplied in a MX genio file.

All index contours must be labelled as Z strings with all other contours being labelled with 0 (zero) strings.  The following table denotes index contour intervals;

| MAPPING CLASS | INDEX CONTOUR INTERVAL | CONTOUR INTERVAL |
| --- | --- | --- |
| A B C | 2 m 5 m 10 m | 0.5 m 1.0 m 2.0 m |

## Detail strings

Full detail is required on all mapping projects.  The topology of the natural surface plus all man made features must be collected.

When detail strings are captured, all common point features must be grouped into a single string label.

### Road capture

All sealed and unsealed roads must be defined by the edges of carriageways and the centreline of the road.  All captured strings on a road shall be 3D.

Individual spot heights points (Main Roads code PRL*) are not to be captured on road surfaces.

Points on a road string must be no more than the distances listed within the table 11.6, unless the string passes under a bridge.  All points captured on a road must adhere to the hard surface accuracies listed in “11.4 Mapping Accuracy”.

### Road markings

Road markings must be captured for class A mapping.  All white line road markings (including internal chevron lines) and turning arrows must be captured in accordance with codes listed in Appendix A and are 2D in nature (i.e. have -999.000 null values).

Mapping class B and C do not require road markings to be captured.

### Bridges

When a road passes under a bridge the captured road strings must be continuous.  Points captured on the road strings passing under the bridge must be as near as possible either side of the bridge to minimize the string length under the bridge.  No ground points under the bridge are to be captured and no soffit levels are required unless specifically requested.

Mapping contours are to be produced on the surface of bridges.  All embankment contours to be edited and clipped at the intersection of a bridge.  No contours are to be produced under bridges.

The centreline of expansion joints are to be captured on all bridges. (Main Roads code EX**).

### Watercourses and High Water Marks

Watercourse features must be three dimensional directional strings.  The thalweg (lowest point) of a stream must be captured where possible in conjunction with break lines and spot heights to accurately define the watercourse shape.  If visible, the high water mark (or flood level debris) must be captured.  (Main Roads code PFL* for points or WJ** for strings.)

### Vegetation

Individual trees are not required to be captured unless specifically requested.  The outer edge of vegetation information shall be captured using the GT (tree line) feature code.

In densely vegetated areas, all visible ground detail (i.e. MRWA code PRL*) must be captured.

### Circular and Square Manholes for Class A Mapping

Man holes captured under class A must be defined by the outer border of the man hole.  NOT the internal lid/cover.  The man hole must be captured as a closed linear feature.  That is, the first and last point of the string is to be the same point. (Fig A.)

If a circular manhole in the field does not have a rectangular concrete border then the perimeter of the circular man hole shall be defined by sufficient points in the data file to create a smooth circle when it is drawn. (Fig B)

Fig A

X – Denotes survey point to be captured.

Fig B

### Circular and Square Manholes for Class B & C Mapping

Circular and square manholes are to be defined by a single point.  The point shall be located in the centre of the lid/cover to define the feature.

### Power Lines

Overhead power lines must be captured as 2D strings (Main Roads code EA**).  Individual wires are not to be mapped.  A single EA** string must denote the location of power lines between poles and or structures (such as buildings).  Power lines must be captured to the edge of the requested survey boundaries.

### Power Poles / Telegraph poles

The base of power and telegraph poles are to be captured as 3D points.

### Light poles

All light poles lining freeways are Main Roads controlled lights and must be coded with the linear code EM (single electric light pole).  All remaining lights within suburbs shall be coded with EL (single electric light pole), LM (multiple electric light pole) or PLP (single light on light pole).

### High Tension Power Towers

High tension power towers shall be captured at ground height by a closed string (Main Roads code EP**) and are to bound the outer edges of the tower feet at ground height.

If the power line structure is built on a concrete block then the concrete structure must additionally be captured (MRWA code for concrete SE or SF) under the high tension power tower.

### Side Entry Pits for Class A Mapping

Side entry pits shall be collected in accordance with the diagram below.  The outer extent of the concrete shall be collected and not the inner concrete lid. (Main Roads code DX00).

## Mapping Codes

All mapped features must be defined by Main Roads codes (Appendix A).  All codes must be compatible with Main Roads MX design software and be a maximum of 4 digits.  All codes starting with “P” are treated as points.  All other codes (except text) become lines and are treated as break lines by Main Roads software when triangulating.

If a mapping code does not exist for a specific feature, then Main Roads requires the feature to be captured using an existing “undefined” code and accompanying it with descriptive text.  As a last preference, and on request, a new feature code can be created by the Data Manager of Main Roads.  All new feature codes must be clearly noted in the survey report.

### Main Roads mapping code table.

Appendix A lists all Main Roads codes, defines them as 2D or 3D points, allocates which class relates to which code (A, B or C) and specifies any other special requirements for codes (i.e. Linear Features or special 3D).

The codes for mapping and digital ground surveys are the same.  However some codes will not be relevant to mapping.

### MX code compression.

Main Roads has developed a string compression program that ensures that the number of captured strings for a single code does not out number the number of code labels available.

This ensures that a single string label does not change into a different code when being run into MX.

All genio files must be run through the MX code compression software before being supplied to Main Roads.  A note must be made within the mapping report that the MX code compressor has been run on all supplied MX genio files.

The free MX code compression software is available on request from the Data Manager within Main Roads.

### 2D and 3D codes.

All 2D points must have null (-999.000) heights attached.  Under no circumstances is a single feature to be supplied as a mixture of 3D and 2D points (i.e. measured heights and        -999.000 values).  If any Main Road code can not be captured as listed in Appendix A (i.e. a 3D point needs to be captured in 2D) then a note must be made in the mapping report to highlight this issue.

### “Special 3D” codes.

Special 3D codes relates to the format of presenting contours, Z and 0 (zero) labels in a MX genio file.  This consists of grouping strings with the same height under a single contour label.  The AHD height is only stated once in the label header and not repeated throughout the coordinate body of the data file.

### Linear Feature Codes

Linear features are Main Roads codes that have symbology attached to them and need to be captured in a specific way.  Linear feature codes are highlighted in Appendix A code list by observing the “code type” column.

The features defined below will be represented in Main Roads software by dynamic symbols and shall be defined by the points indicated by ‘X’ in the diagrams for the specific feature. The nature of these features and/or the method of definition may not provide a true representation of the surface area, and as such, features of this nature will be classed as non-contourable (2D Strings).

#### Centrally Located Linear Features

A number of linear features are defined by a string running through the centre of the feature.

The strings denoting the features have symbology automatically attached to them when they are run into MX by Main Roads planners and designers.

RX01					       GH01

Cross Walk				    	      Hedge

X’s represent capture points.

#### Electric Light Poles

MX Codes LM01, LR01, EL01 and EM01.

These codes represent multiple and single electric lights on a pole.  These are defined by a first measurement (1) at the centre of the pole post with a second measurement (2) under the over hanging light.  In the case of a multiple light the second measurement is taken towards one light only.

Multiple light electrical pole   	       	     Single light electrical pole

code LM (Western Power) and LN (MRWA)	           codes EL (Western Power) and EM (MRWA)

All light poles lining the freeway, bridges and control of access highways are lights owned and controlled by MRWA (LN01 and EM01).

#### Pavement Arrows

MX Code ARØ1, ASØ1, ATØ1, AUØ1, AVØ1, AWØ1 AXØ1 and AY01

These codes represent various pavement turning arrows. These are defined by a first measurement (1) at the start or base of the arrow with a second measurement (2) taken along the axis of the arrow at its point or extremity.

#### Left and Right Handed Features

Certain features are described by left and right codes.

E.g.	Code SZØ1 BUILDING (RIGHT)

Code SYØ1 BUILDING (LEFT)

These different codes define what side of the feature its symbol is plotted.  If the feature is defined by two points the feature is either on the left or right of the points in the order they are being surveyed.  In the example below, two codes are used to define two walls of a building to show the different symbology associated with the code.  Please note, only one continuous code should be used to capture an entire building.

## Text Data

Text / annotation is required within all mapping.

At a minimum the following features must be labelled with text

- Road names.

- Rivers, water bodies and water courses.

- Bridge names if known.

- Names of survey control if used – SSM’s, RRM’s, BM’s

- Out of tolerance areas

- Major features – hospitals, shopping centres etc

- Provide additional information on features – i.e. fence types if requested etc.

Text shall not be placed on spot heights (Main Roads code PRL) or contours.

## Mapping Extent String

An outline of the mapping extent must be supplied within the mapping data file.  This string must be labelled as a two dimensional “B” string.  The string must form a closed polygon around the mapped area.

## Features not required to be captured.

Guide posts on the verge of the road.

## Verification Survey.

An independent verification survey is required to be performed by the contractor on all mapping projects.  Class A mapping requires a “Verification String Survey (11.12.1)”.   Class B and C mapping will require a “Verification Cross Survey (11.12.2)”.  The contractor will statistically analyse the verification survey with the produced mapping and provide a verification statement. (See “11.12.3 Verification Statement”.)  The mapping report must confirm the compliance of the mapping with the requested accuracy.  A template mapping report is provided as Appendix B.

The verification survey capture must be an independent survey that is performed by a method other than mapping.  Additionally, the verification survey must be of a higher accuracy than the requested mapping.  Ideally the accuracies required for verification strings will be:

± 0.03m horizontally and ± 0.03m vertically.

The verification strings must endeavor to capture varying features and terrains (i.e. not all on the bitumen road) and ideally must follow the proposed alignment of the future road as demonstrated in “11.12.1 Verification String Survey” and “11.12.2 Verification Cross Survey”.

The strings must have sufficient suitable points to define and follow the ground surface and changes of grade.   The verification strings will additionally, where possible, capture detail (i.e. corner of fences, power poles, road edge etc) to compare horizontal and vertical accuracies of detail within the mapping model.

Verification survey string data is required to be lodged as a separate data file.  It is not to be included within the mapping MX genio file.

### Verification String Survey – Class A.

A verification string survey must be used to verify class A mapping.  A verification string survey must consist of a series of three dimensional points that are captured along the entire length of the mapping corridor defining the ground shape.  Ideally the verification string is to cover the proposed alignment of the future road.  This alignment can be provided by the contract manager.

In addition to the continuous verification string there are to be cross section strings.  All cross sections must be perpendicular to the continuous verification string and must be a minimum of a third of the width of the mapping corridor.  There must be a minimum of three cross sections per mapping project or cross sections no more than seven kilometers apart.

In the diagram below, an example of verification strings are indicated as orange lines within the mapping corridor.

### Verification Cross Survey – Class B and C.

The verification cross survey must be used to verify class B and C mapping.

A verification cross consists of two lines (strings) of three dimensional points defining the ground shape perpendicular to each other to form crosses.  Ideally, each cross is to cover the proposed area of the future road design on the ground.  Cross sections are to be no more than ten kilometers apart.  Regardless of project extents, a minimum of three cross sections is required per mapping project.

Verification crosses must be a minimum of one-third the width of the mapping corridor.

An example of verification strings are indicated as orange lines in the diagram below.

### Statistical analysis and verification statement.

The verification strings and the mapping model are to be statistically analyzed using a normal distribution curve.

The consusltant shall compare the verification statistics with the requested mapping accuracy class.  To achieve this, separate statistic values will need to be generated for hard and soft surfaces.

The minimum type of statistics required to be presented on each accuracy requirement are the;

Number of audit points

Mean

Median

Standard Deviation

Range

Minimum value

Maximum value

The contractor shall produce the above statistics and present a comparison to the relevant accuracy requirements stated in 11.4.1 and 11.4.2.

All outlier’s greater than three standard deviations must be investigated to determine if it is an error in the mapping model or a blunder.

If blunders are identified and confirmed, then these values must be removed and the statistics rerun to produce new statistics.  The original statistics, blunders and justification of removing the point must be noted within the report.

An example of statistical analysis and a verification statement are presented in Appendix C Mapping Report Example.

## Verification Survey String Data File

The verification data must consist of all strings used to produce the statistics in the verification statement.  String label must be VS**

# FINAL data supply.

All mapping data and reports must be lodged with the Data Manager or Senior Mapping Surveyor, Road and Traffic Engineering Branch, Main Roads.

The following materials must be presented to Main Roads with each mapping project:

- Mapping report

- A MX Genio mapping data file in the local Main Roads coordinate system

- A MX Genio verification string data file.

- Mapping base material (i.e. diapositives, scan, contacts, control information etc)

## Mapping and Verification Report.

A mapping report must be supplied with the MX Genio mapping data files.

All mapping reports must be lodged as a .doc (Microsoft word) and a Pdf file.

The mapping report title must match the data files and make reference to the fact that it is a mapping report.  For example

H6_SLK_25_55_Mapping Report.doc

A mapping report template is located as Appendix B.

## Naming MX Genio files.

The requested mapping area will be provided to Main Roads in a compressed digital file and must be labelled only with alphabetic, numeric or underscore characters (i.e. symbols are not to be used within the name).  The file is to have a logical name that is associated with the mapping project and must end in the extension .gen.

For example, if the Great Northern Highway (known as H6 within Main Roads) was mapped with a corridor from SLK 25 – 55 then the data files would be labelled:

H6_SLK_25_55_HUG94.gen

H6_SLK_25_55_AUDIT_HUG94.gen

Road numbers and SLK’s can be provided by the contract manager.

## Mapping Base Material.

At the completion of the mapping project all controlled contacts, controlled diapositives, control details, electronic scans or any other information pertaining to the Main Roads project must be supplied to the Senior Mapping Surveyor, Road and Traffic Engineering Branch, Main Roads.

# MERGING MAPPING AND DIGITAL GROUND SURVEY DATA.

If Main Roads requests a survey of an area using a combination of mapping and ground survey techniques, then the data must result in a seamless complete homogeneous 3D model with all detail captured.  There must be no feature captured twice or features missing between the adjoining mapping and ground survey data sets.

When a combination of mapping data and ground survey data are collected then the following must apply:

- The mapping data must conform to the Mapping standard 67-08-44 and the ground survey must adhere to the Digital Ground Survey Standard 67-08-43.

- The mapping and ground survey data must NOT be supplied as a single combined file.  Separate files must be supplied for the mapping and the ground survey.

- The mapping data files must contain holes (no data) where the ground survey has been performed within the requested survey area.

- The digital ground survey must include headwalls, culverts, drains, road formation (i.e. shoulder, drain and road surface).

- All mapping contours are to be cut from the ground survey data area.

- No data feature shall appear in both the mapping and ground survey unless they are butt joining points for strings.

- All string features changing from mapping to ground survey must be seamless with strings joining.  The join point must be the point captured by the Digital Ground Survey. (I.e. the higher order ground survey point included in the lower order mapping data.)   String features must not overlap.  I.e. a single string can only contain one common point when passing from the mapping survey to the ground survey.

- Individual spot heights (Main Roads code PRL) are not to be captured on the road surfaces.

- Where an island occurs between traffic lanes all ground features must be captured and must include natural surface points (PRL’s).

- Separate meta data sheets are to be supplied for the mapping and the ground survey and both are to be included in the final report.

- Individual boundary strings are to be supplied for both the mapping and the ground survey areas.  These must denote the outline of the area of each survey technique.

# SUPPLY MEDIUMS FOR DIGITAL DATA.

Digital Data must be presented on either of the following mediums:

CDs			-	CD-R or CD-RW

-	700 MB

  - Main Roads Windows compatible (Windows XP)

and / or

DVDs				-	 DVD-R or DVD-RW

-	4.7 GB

-	Main Roads Windows compatible (Windows XP)

Files may be supplied in compressed format provided they are self extracting.

# APPENDIX A – Main Roads feature codes.

Definitions

2D - two dimensional capture (E, N).

3D - three dimensional capture (E, N and height).

SPECIAL 3D - A special three dimensional string format to save space within the MX ascii genio data file.

CLASS - codes required to be captured under requested mapping class.

LINEAR FEATURE CODE - special capture requirements for Main Roads symbology. Please see section 11.8.5.

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| * | TEXT STRINGS | 2D |  | A, B & C |  |
| AM | AMCOM CABLE |  |  |  |  |
| AR | ARROW | 2D |  | A | LINEAR FEATURE CODE |
| AS | ARROW LEFT AND STRAIGHT | 2D |  | A | LINEAR FEATURE CODE |
| AT | ARROW RIGHT AND STRAIGHT | 2D |  | A | LINEAR FEATURE CODE |
| AU | ARROW LEFT | 2D |  | A | LINEAR FEATURE CODE |
| AV | ARROW RIGHT | 2D |  | A | LINEAR FEATURE CODE |
| AW | ARROW LEFT RIGHT & STRAIGHT | 2D |  | A | LINEAR FEATURE CODE |
| AX | ARROW L & R | 2D |  | A | LINEAR FEATURE CODE |
| AY | ARROW U TURN | 2D |  | A | LINEAR FEATURE CODE |
| B | MAPPING EXTENTS | 2D |  | A, B & C |  |
| BC | BARRIER, CONCRETE |  | 3D | A & B |  |
| BI | BIKE RACK | 2D |  | A |  |
| BP | BRICK PAVING |  | 3D | A & B |  |
| BS | PARK BENCH / SEATING | 2D |  | A & B |  |
| BT | GUARDRAIL - THRIE |  | 3D | A & B |  |
| BW | BARRIER STEEL WIRE ROPE |  | 3D | A & B |  |
| D | DRAIN |  | 3D | A, B & C |  |
| DA | SEWER PRESSURE MAIN |  | 3D | A, B & C |  |
| DC | CULVERT |  | 3D | A, B & C |  |
| DD | EDGE OF DRAIN (LEFT) |  | 3D | A & B |  |
| DE | EDGE OF DRAIN (RIGHT) |  | 3D | A & B |  |
| DF | FLOODWAY |  | 3D | A, B & C |  |
| DG | GULLY |  | 3D | A |  |
| DH | HEADWALL |  | 3D | A |  |
| DL | LGA DRAINAGE MANHOLE |  | 3D | A |  |
| DM | SUMP (LEFT) |  | 3D | A |  |
| DN | SUMP (RIGHT) |  | 3D | A |  |
| DP | DRAINAGE PIPE |  | 3D | A |  |
| DR | MAIN ROADS DRAINAGE MANHOLE |  | 3D | A |  |
| DS | SEWERAGE MANHOLE |  | 3D | A |  |
| DT | TRAFFIC DETECTOR LOOP |  | 3D | A |  |
| DU | UNDEFINED MANHOLE |  | 3D | A |  |
| DV | SEWERAGE PIPE |  | 3D | A, B & C |  |
| DW | WATER CORP DRAINAGE MANHOLE |  | 3D | A |  |
| DX | SIDE ENTRY PIT |  | 3D | A |  |
| DZ | RAIL CABLE MANHOLE |  | 3D | A |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| EA | OVERHEAD POWER LINES | 2D |  | A & B |  |
| EG | GAS MANHOLE |  | 3D | A |  |
| EH | GAS LINE |  | 3D | A, B & C |  |
| EK | GAS CYLINDER / TANK | 2D |  | A & B |  |
| EL | ELECTRIC LIGHT POLE | 2D |  | A & B | LINEAR FEATURE CODE |
| EM | MAIN ROADS ELECTRIC LIGHT POLE | 2D |  | A & B | LINEAR FEATURE CODE |
| EP | HIGH TENSION POWER TOWER |  | 3D | A, B & C |  |
| ET | TRANSFORMER (DOUBLE) |  | 3D | A |  |
| EU | MAIN ROADS ELECTRIC MAN HOLE |  | 3D | A |  |
| EX | EXPANSION JOINT ON BRIDGE |  | 3D | A & B |  |
| F | FENCE OTHER |  | 3D | A, B & C |  |
| FC | KOPPA LOGGING FENCE |  | 3D | A & B |  |
| G | UNDEFINED FEATURE |  | 3D | A, B & C |  |
| GA | RECREATION AREA |  | 3D | A, B & C |  |
| GB | LEVEE BOTTOM |  | 3D | A, B & C |  |
| GC | LAWN |  | 3D | A, B & C |  |
| GD | GARDEN BED |  | 3D | A, B & C |  |
| GE | EARTHWORKS AREA |  | 3D | A, B & C |  |
| GF | FOOTPATH |  | 3D | A & B |  |
| GG | GATE |  | 3D | A, B & C |  |
| GH | HEDGE |  | 3D | A, B & C | LINEAR FEATURE CODE |
| GI | MINE WORKINGS |  | 3D | A, B & C |  |
| GJ | GROUND SUBSIDENCE |  | 3D | A, B & C |  |
| GK | VENT SHAFT |  | 3D | A, B & C |  |
| GL | LEVEE TOP (LEFT) |  | 3D | A, B & C |  |
| GM | LEVEE TOP (RIGHT) |  | 3D | A, B & C |  |
| GN | BORROW PIT (LEFT) |  | 3D | A, B & C |  |
| GO | MINE SHAFT |  | 3D | A, B & C |  |
| GP | BORROW PIT (RIGHT) |  | 3D | A, B & C |  |
| GQ | ROCK OUTCROP (LEFT) |  | 3D | A, B & C |  |
| GR | ROCK OUTCROP (RIGHT) |  | 3D | A, B & C |  |
| GS | BUSH LINE | 2D |  | A, B & C |  |
| GT | TREE LINE/CANOPY(.1.5m) | 2D |  | A, B & C |  |
| GU | ORCHARD |  | 3D | A, B & C |  |
| GV | VINEYARD |  | 3D | A, B & C |  |
| GW | PLANTATION |  | 3D | A, B & C |  |
| GX | RAIL BOOM GATE |  | 3D | A, B & C |  |
| GY | MARKET GARDEN |  | 3D | A, B & C |  |
| GZ | NURSERY |  | 3D | A, B & C |  |
| HB | HYDRANT BOOSTER BOX | 2D |  | A & B |  |
| HD | SHED | 2D |  | A & B |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| K | CADASTRAL BOUNDRY | 2D |  | A, B & C |  |
| L | BREAKLINE |  | 3D | A, B & C |  |
| LM | MULTIPLE LIGHT POLE (Western Power)) | 2D |  | A & B | LINEAR FEATURE CODE |
| LN | MULTIPLE LIGHT POLE (MRWA) | 2D |  | A & B | LINEAR FEATURE CODE |
| LR | RAIL SIGNAL BOX |  | 3D | A |  |
| N | LINE OF NATURAL SURFACE LEVELS |  | 3D | A, B & C |  |
| NB | BANK BOTTOM |  | 3D | A, B & C |  |
| NR | RIDGE |  | 3D | A, B & C |  |
| NT | BANK TOP (LEFT) |  | 3D | A, B & C |  |
| NU | BANK TOP (RIGHT) |  | 3D | A, B & C |  |
| PAC | ROAD REFERENCE MARK |  | 3D | A |  |
| PAN | ANTENNA | 2D |  | A, B & C |  |
| PBB | GRASS TREE |  | 3D | A |  |
| PBF | FUEL BOWSER | 2D |  | A, B & C |  |
| PBH | BOREHOLE | 2D |  | A & B |  |
| PBL | BOLLARD | 2D |  | A & B |  |
| PBM | BENCHMARK |  | 3D | A |  |
| PBS | BUS STOP FINGER POLE | 2D |  | A & B |  |
| PBU | BUSH | 2D |  | A, B & C |  |
| PCA | CONTROL OF ACCESS SIGN |  | 3D | A |  |
| PCC | CONSTRUCTION CONTROL POINT |  | 3D | A & B |  |
| PCM | MRWA CABLE MARKER |  | 3D | A & B |  |
| PCT | TELEPHONE TOWER (CENTRE) | 2D |  | A, B & C |  |
| PDL | DECK LEVEL |  | 3D | A & B |  |
| PDM | DIEBACK AREA MARKER |  | 3D | A |  |
| PDT | PETROLEUM TANK | 2D |  | C |  |
| PEC | SEC ELECTRIC CABLE BOX |  | 3D | A & B |  |
| PED | ELECTRICAL POWER DOME |  | 3D | A & B |  |
| PEP | EMERGENCY PHONE |  | 3D | A & B |  |
| PET | TRANSFORMER (SINGLE) | 2D |  | A |  |
| PFB | TELEPHONE BOOTH | 2D |  | A & B |  |
| PFG | FLAGPOLE | 2D |  | A & B |  |
| PFI | FLOOD LEVEL INDICATOR |  | 3D | A |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| PFL | HIGH WATER MARK POINT |  | 3D | A, B & C |  |
| PFP | UNDERGROUND TANK FILLER |  | 3D | A & B |  |
| PFR | FLOOR LEVEL |  | 3D | A & B |  |
| PFS | FINGER SIGN |  | 3D | A |  |
| PGF | GROUND FLOOD LIGHT |  | 3D | A & B |  |
| PGG | POINT GATE |  | 3D | C |  |
| PGM | GAS MARKER |  | 3D | A & B |  |
| PGU | GAS TEST VALVE |  | 3D | A |  |
| PGV | GAS VALVE |  | 3D | A |  |
| PHG | HYDRANT (GROUND) |  | 3D | A & B |  |
| PHL | HIGH MAST LIGHTING |  | 3D | A & B |  |
| PHP | HYDRANT (PILLAR) |  | 3D | A & B |  |
| PHT | HIGH TENSION POWER POLE |  | 3D | A, B & C |  |
| PHU | HIGH TENSION PYLON CENTRE |  | 3D | C |  |
| PIA | ILLUMINATED ADVERTISING SIGN |  | 3D | A & B |  |
| PIS | SIGN ONE POLE |  | 3D | A & B |  |
| PJB | TRAFFIC PAVEMENT JUNCTION BOX |  | 3D | A & B |  |
| PLB | LITTER BIN |  | 3D | A & B |  |
| PLP | SINGLE LIGHT ON A SINGLE POLE |  | 3D | A & B |  |
| PLW | WATER LEVEL |  | 3D | A, B & C |  |
| PMB | MAIL BOX |  | 3D | A & B |  |
| PML | LGA MANHOLE CENTRE |  | 3D | C |  |
| PMM | MINE SHAFT |  | 3D | C |  |
| PMR | MRWA DRAINAGE MAN HOLE CENTRE |  | 3D | C |  |
| PMS | SEWER MANHOLE CENTRE |  | 3D | C |  |
| PMT | TELEPHONE MAN HOLE CENTRE |  | 3D | C |  |
| PMU | UNDEFINED MAN HOLE CENTRE |  | 3D | C |  |
| PMW | WAWA DRAINAGE MAN HOLE CENTRE |  | 3D | C |  |
| PMX | RAIL CABLE MAN HOLE CENTRE |  | 3D | C |  |
| PPB | TELEPHONE PIT OR BOX |  | 3D | A & B |  |
| PPC | PHOTO TRIANG. POINT |  | 3D | A, B & C |  |
| PPE | MAIN ROADS ELECTRIC SUPPLY POLE |  | 3D | A & B |  |
| PPG | PEG |  |  |  |  |
| PPH | PHOTO TARGET CONTROL POINT |  | 3D | A, B & C |  |
| PPM | PARKING METER |  | 3D | A |  |
| PPP | POWER POLE |  | 3D | A & B |  |
| PPR | RAIL TELEPHONE BOX |  | 3D | A & B |  |
| PPS | PEDESTRIAN SIGN |  | 3D | A |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| PPT | TELEPHONE POLE |  | 3D | A & B |  |
| PRB | RAIL CABLE PIT OR BOX |  | 3D | C |  |
| PRC | RAIL TRAFFIC CONTROL BOX |  | 3D | C |  |
| PRG | BOOM GATE |  | 3D | C |  |
| PRK | SLK POST |  | 3D | A |  |
| PRL | NATURAL SURFACE SPOT HEIGHT |  | 3D | A, B & C |  |
| PRS | RAIL TRAFFIC SIGNALS |  | 3D | A, B & C |  |
| PRT | RETICULATION SPRINKLER |  | 3D | A |  |
| PRX | RAIL CABLE MARKER |  | 3D | A |  |
| PSA | SEWER INSPECTON SHAFT |  | 3D | A |  |
| PSC | SECWA UNDER GRND CABLE MARKER |  | 3D | A |  |
| PSD | STOCK TROUGH | 2D |  | A & B |  |
| PSM | STANDARD SURVEY MARK |  | 3D | A, B & C |  |
| PSN | TRIG POINT |  | 3D | A, B & C |  |
| PSP | STAY POLE |  | 3D | A & B |  |
| PST | STANDPIPE | 2D |  | A & B |  |
| PSU | RETIC. CONTROL VALVE |  | 3D | A |  |
| PSV | STOP VALVE |  | 3D | A |  |
| PSW | SEWER VENT |  | 3D | A |  |
| PSX | SEWER LINE MARKER |  | 3D | A |  |
| PTB | TEMPORARY BENCH MARK |  | 3D | A, B & C |  |
| PTC | TRAFFIC CONTROLLER |  | 3D | A |  |
| PTE | TRAFFIC EARTH PIT |  | 3D | A & B |  |
| PTH | TICKET MACHINE |  | 3D | A |  |
| PTK | TANK | 2D |  | C |  |
| PTL | TREE (0.3-1.0m dia.) |  | 3D | A & B |  |
| PTM | TELEPHONE MARKER |  | 3D | A & B |  |
| PTP | TELEPHONE PILLAR |  | 3D | A & B |  |
| PTR | TREE (1.0 -1.5m dia.) |  |  | A & B |  |
| PTS | TRAFFIC SIGNALS 1 ASPECT |  | 3D | A & B |  |
| PTT | TRAFFIC SIGNALS 2 ASPECTS |  | 3D | A & B |  |
| PTU | TRAFFIC SIGNALS 3 ASPECTS |  | 3D | A & B |  |
| PTV | TRAFFIC SIGNALS 4 ASPECTS |  | 3D | A & B |  |
| PTW | TREE (0 – 0.3m dia.) |  | 3D | A & B |  |
| PTX | POLICE TRAFFIC CAMERA | 2D |  | A & B |  |
| PWA | STEEL WIRE ANCHOR |  | 3D | A & B |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| PWB | WATER BORE |  | 3D | A & B |  |
| PWD | WINDMILL |  | 3D | A, B & C |  |
| PWL | WELL |  | 3D | A, B & C |  |
| PWM | WATER METER |  | 3D | A |  |
| PWR | WATER MAIN MARKER |  | 3D | A & B |  |
| PWT | TAP |  | 3D | A & B |  |
| PWV | AIR VALVE |  | 3D |  |  |
| PZM | PEIZOMETER |  | 3D |  |  |
| R | ON ROAD |  | 3D | A & B |  |
| RB | KERB FRONT BOTTOM |  | 3D | A & B |  |
| RC | CENTRE OF ROAD |  | 3D | A, B & C |  |
| RD | EDGE OF DRIVEWAY |  | 3D | A & B |  |
| RE | EDGE OF BITUMEN UNKERBED |  | 3D | A & B |  |
| RF | PARKING BAY / REST AREA |  | 3D | A, B & C |  |
| RG | CATTLE GRID |  | 3D | A & B |  |
| RH | OVERHEAD SIGN | 2D |  | A & B |  |
| RI | DOUBLE BARRIER LINE MARKING |  | 3D | A | LINEAR FEATURE CODE |
| RJ | OVERTAKING LANE LEFT |  | 3D | A | LINEAR FEATURE CODE |
| RK | TOP OF KERB REAR |  | 3D | A & B |  |
| RL | RAIL (CENTRE OF TRACK) |  | 3D | C |  |
| RM | LINE MARKING BROKEN |  | 3D | A |  |
| RN | SIGN MULIPLE POLES |  | 3D | A & B |  |
| RO | OVERTAKING LANE RIGHT |  | 3D | A | LINEAR FEATURE CODE |
| RP | PEDESTRIAN RAMP |  | 3D | A |  |
| RQ | TURNING LANE (1M line with 1M gap) |  | 3D | A |  |
| RR | SINGLE SOLID LINE |  | 3D | A |  |
| RS | SHOULDER |  | 3D | A & B |  |
| RT | TRACK |  | 3D | A & B |  |
| RU | EDGE OF UNSEALED ROAD |  | 3D | A & B |  |
| RV | TURNING LANE (1M line with 1M gap) |  | 3D | A |  |
| RW | AERIAL ROAD SPEED MARKER |  | 3D | A | LINEAR FEATURE CODE |
| RX | CROSSWALK | 2D |  | A | LINEAR FEATURE CODE |
| RY | RAILWAY LINE (ON TRACK) |  | 3D | A & B |  |
| RZ | RAILWAY PLATFORM |  | 3D | A & B |  |
| SA | ABUTMENT |  | 3D | A, B & C |  |
| SB | BUS SHELTER |  | 3D | A, B & C |  |
| SC | COLUMNS |  | 3D |  |  |
| SD | BRIDGE |  | 3D | A, B & C |  |
| SE | EDGE OF CONCRETE LEFT |  | 3D | A, B & C |  |
| SF | EDGE OF CONCRETE RIGHT |  | 3D | A, B & C |  |
| SG | GUARDRAIL | 2D |  | A, B & C |  |
| SI | MEMORIAL |  | 3D | A, B & C |  |

APPENDIX A

| CODE | DESCRIPTION | 2D | 3D | CLASS | CODE TYPE |
| --- | --- | --- | --- | --- | --- |
| SJ | WALL (LEFT) |  | 3D | A, B & C |  |
| SK | WALL (RIGHT) |  | 3D | A, B & C |  |
| SL | RETAINING WALL (LEFT) |  | 3D | A & B |  |
| SM | RETAINING WALL (RIGHT) |  | 3D | A & B |  |
| SN | RUIN |  | 3D | A, B & C |  |
| SO | FOOT BRIDGE |  | 3D | A, B & C |  |
| SP | PIER | 2D |  | A, B & C |  |
| SR | RAMP |  | 3D | A, B & C |  |
| SS | STEPS OR STAIRS | 2D |  | A, B & C | LINEAR FEATURE CODE |
| ST | CONVEYOR | 2D |  | A, B & C |  |
| SU | UNDERPASS |  | 3D | A, B & C |  |
| SV | VERANDAH | 2D |  | A, B & C |  |
| SW | AWNING (LEFT) | 2D |  | A, B & C |  |
| SX | AWNING (RIGHT) | 2D |  | A, B & C |  |
| SY | BUILDING (LEFT) | 2D |  | A, B & C |  |
| SZ | BUILDING (RIGHT) | 2D |  | A, B & C |  |
| VC | TELEPHONE CABLE |  | 3D | A, B & C |  |
| VM | TELEPHONE MANHOLE |  | 3D | A & B |  |
| VS | VERIFICATION STRING |  | 3D | A, B & C |  |
| VT | TELEPHONE TOWER |  | 3D | A, B & C |  |
| WA | WET AREA |  | 3D | A, B & C |  |
| WB | CREEK (UPSTREAM) |  | 3D | A, B & C |  |
| WC | CREEK (DOWNSTREAM) |  | 3D | A, B & C |  |
| WD | DAM (LEFT) |  | 3D | A, B & C |  |
| WE | DAM (RIGHT) |  | 3D | A, B & C |  |
| WF | EDGE OF CREEK/RIVER (RIGHT) |  | 3D | A, B & C |  |
| WG | EDGE OF CREEK/RIVER (LEFT) |  | 3D | A, B & C |  |
| WH | SWIMMING POOL |  | 3D | A, B & C |  |
| WJ | FLOOD LEVEL LINE |  | 3D | A, B & C |  |
| WK | EDGE OF WATER (RIGHT) |  | 3D | A, B & C |  |
| WL | EDGE OF WATER (LEFT) |  | 3D | A, B & C |  |
| WM | CL OF CHANNEL D/STREAM |  | 3D | A & B |  |
| WN | CL OF CHANNEL UPSTREAM |  | 3D | A & B |  |
| WP | WATERPIPE |  | 3D | A & B |  |
| WS | SWAMP |  | 3D | A, B & C |  |
| WT | TANK PERIMETER | 2D |  | A, B & C |  |
| X | SPECIFIED WATERWAY SECTIONS |  | 3D | A, B & C |  |
| XX | OUT OF TOLERANCE STRING |  | 3D | A, B & C |  |
| Z | INDEX CONTOURS |  | Special 3D | A, B & C |  |
| 0 | CONTOURS |  | Special 3D | A, B & C |  |

# APPENDIX B – Mapping report example

MAPPING   REPORT

PROJECT TITLE:  Cut / paste from Main Roads request.

Survey Panel Contract No / ETS No: Cut / paste from Main Roads request.

Date mapping submitted: The date the mapping and this report is lodged at Main Roads.

This report was compiled by:  John Smith

Senior Photogrammetrist

Phone number

This work was performed by: Your company logo

Address

Project Brief.

Cut / paste from Main Roads request – “project brief”.

Methodology

A brief outline can take on the following point form –

- X photo control points were coordinated. (see flight diagram)

- X runs were flown and Y frames were exposed.  (see flight diagram)

- A film/digital (name i.e. RC30) camera was used.

- Photography was registered with Landgate and the Registration number is: 123456.

- Triangulation was performed with X software.

- The residuals on the points were X, Y and Z RMS.

- Photo control points were horizontally surveyed using static GPS

- Photo control points were vertically surveyed to achieve 12 root K accuracies using level and staff.

- Aerial photography was scanned at X microns producing a pixel size of Y meters on the ground

- X models were mapped to achieve class X mapping.

- Mapping was captured using (name of) work stations.

- Contours were directly observed or Contours were automatically generated.

- Verification points were captured using RTK / total station.

- Verification point layout shown in attached diagram X

- Audit statistics were generated using excel spread sheet / X software.  Details of the verification survey are outline in the Verification Survey Analysis section.

- Mapping data was edited with ground survey data and gen files generated.

The following subcontractors were used:

XX to fly photography.

Please note: The shorter this section the better.  You can adopt or be as brief as the points above.

Project issues.

Two areas of thick impenetrable vegetation were encountered within the mapping project area.  These have been outlined by XX00 strings.  Intermittent points were able to be observed within the vegetation.  Approximate accuracy of the out of tolerance areas is -+0.01m.

The following features were not found in the Main Roads mapping code list.   After contacting the Main Roads Data Manager new codes were allocated for the following features.

These are;

XXXX – for y structures

A number of complaints were received by our surveyors in the field these consisted of -

Our surveyors could not gain entry into the following areas … due to …

Aerial photography was flown on a number of days due to rain …

Cyclone delayed flying of photography and additionally restricted access by our surveyors into areas to lay and coordinate targets.

Data lodged.

The string compression software has been run on the final lodged MX Genio mapping files.

The following data files were supplied for this project

H6_SLK_25_55_Mapping Report.doc

H6_SLK_25_55_PCG94.gen

H6_SLK_25_55_AUDIT_ PCG94.gen

Verification survey analysis and report.

A cross/string verification survey was performed on the mapping.  The verification strings were surveyed by total station / RTK GPS.

Class B accuracy was requested

|  | HARD SURFACES |  | SOFT SURFACES |  |
| --- | --- | --- | --- | --- |
| MAPPING CLASS | VERTICAL | HORIZONTAL | VERTICAL | HORIZONTAL |
| B | ± 0.20 m | ± 0.25 m | ± 0.30 m | ± 0.50 m |

The following statistics were generated from our verification survey vs. interpolated mapping points.  The interpolated mapping points were generated using Leica / 12D software.  Statistics on the differences were generate using excel (Data Analysis) / X software.

| Mean | 14.7 |
| --- | --- |
| Median | 12.9 |
| Mode | 57.1 |
| Standard Deviation | 39.84702 |
| Kurtosis | -0.42697 |
| Skewness | 0.316804 |
| Range | 185.5 |
| Minimum | -67 |
| Maximum | 118.5 |
| Sum | 2794.6 |
| Count | 190 |

A number of outlier points were investigated as these fell outside acceptable tolerances.   When checked in the field / compared to SkyView imagery these were found to be very uneven rock outcrops and were isolated points.

When these outliers were removed the statistics were regenerated the following results were produced ….

Any additional comments to be included …

Survey errors considered …

No survey error for GPS audit points have been considered but a value of -/+0.025m would be valid for the RTK methodology.

Summary.

In SUMMARY our resultant statistics are within the required tolerances of the requested mapping.

An individual page shall be used for mapping metadata.  An additional metadata table is required if ground survey if ground survey was requested.

MAPPING METADATA SHEET

| Metadata Category | Core Metadata Element | Description |
| --- | --- | --- |
| Dataset | Title | Photogrammetric Mapping of Great Eastern Highway between SLK 0 - 100 |
|  | Custodian | Survey and Mapping Branch |
|  | MAIN ROADS Contract number | 9999/99 |
|  | Description | 1.5 km wide mapping corridor over Great Eastern Hwy for preliminary design |
| Contact Address | Organisation | John Smith Aerial Mapping |
|  | Contact Position | John Smith |
|  | Mail Address | 99 Smithdale Court East Perth WA 6005 |
|  | Telephone | 9666 6666 |
|  | Facsimile | 9666 6665 |
|  | Electronic Mail | John.smith@inett.com |
| Photography Description | Aerial photography title | Great Eastern Highway SLK - 00 |
|  | Film number | WA 3566 (c) |
|  | Scale |  |
|  | Pixel resolution | 0.05 m |
|  | LANDGATE job number | 059999 |
| Dataset Currency | Date Flown | st Jan, 2000 |
|  | Date Mapped | 2nd Feb, 2000 |
| Dataset Status | Progress | Stage 1 of 2 |
| Access | Mapping Dataset Format | MX |
| Projection | Coordinate System | Perth Coastal Grid |
| Data Quality | Lineage | Joins stage 2 in Hills Upper Grid |
|  | Positional Accuracy | Class A |
|  | Attribute Accuracy |  |
|  | Completeness | Stage three to follow |
| Metadata Date | Metadata Date |  |
| Further Information | Approved non standard codes | None |
|  | Problems encounter | Fred Bloggs refusing entry to land (see attachment) |
|  | Any relevant information or comment |  |
|  | Further Information |  |

ORTHO IMAGE METADATA SHEET

| Metadata Category | Core Metadata Element | Description |
| --- | --- | --- |
| Dataset | Image file name | Road2A.ecw (or Road2A.tif) |
|  | Type of image | Ortho photo/Mosaic/Single Image/Oblique Image |
|  | Title | Great Eastern Highway SLK 1-100 |
|  | Custodian | Mrwa Survey and Mapping Branch |
|  | Main Roads Contract number | 9999/99 if applicable |
|  | Description | 1 km wide Image centred around Great Eastern Hwy. |
| Consultant Contact Address | Organisation | John Smith Aerial Mapping |
|  | Contact Position | John Smith |
|  | Mail Address | 99 Smithdale Court East Perth WA 6005 |
|  | Telephone | 9666 6666 |
|  | Facsimile | 9666 6665 |
|  | Electronic Mail | John.smith@inett.com |
|  | Reference number | Smith 0999 |
| Image Description | Resolution | 0.15M |
|  | Width | 567.1 mm and 6698 pixels |
|  | Length | 938.5 mm and 11085 pixels |
|  | Format | Tif / ecw |
|  | Date of Construction. | 15/3/02 (Not date of photography.) |
|  | Coordinate System | Perth Coastal Grid 94 |
|  | Ground Accuracy or Class |  |
| Source data | Source data | Aerial photography: Great Eastern Highway SLK 1-100 |
|  | Film number | WA 3566 (c) |
|  | Scale | 1:40000 |
|  | DLI job number | 019998 |
|  | Date Flown | 1st Jan, 2001 |
|  | Original scan resolution | 1200 dpi |
| Overlay Information if applicable | Source data | Verified Cadastral Data |
|  | Source | DLI |
|  | Accuracy | 0.2m |
|  | Date of acquisition | 15/1/2001 |
|  | File Format | DGN / ERV / DXF / DWG |
|  | Source data | Topo – DLI 1:25000 digital mapping |
|  | Source | DLI |
|  | Scale | 1:25000 |
|  | Accuracy | 1.5m |
|  | Date of acquisition | 25/5/92 |
|  | File Format | DGN / ERV / DXF / DWG |
|  | Source data | Design |
|  | Source | Main Roads |
|  | Accuracy | 0.1m |
|  | Date of acquisition | 10/1/2001 |
|  | File Format | DGN / ERV / DXF / DWG |
| Further Information | Further Information |  |

Flight Diagram

Mapping Boundary, Audit String and photo control diagram.

Can consist of a screen dump from a CAD package and would preferable a include SkyView background..

These tables for photo control points are only to be supplied on request and generally are only pertinent to urban projects when photo control points have been retained for future use

Final adjusted photo control coordinates

Heights are on the marks left in the ground and not ground / photo target heights.

This is to be in table form supplied in XXX local grid and MGA ZXX grid.

.

XXX Local grid

| Point number | Easting | Northing | Height |
| --- | --- | --- | --- |

Property Contact List

| Target Number | Property Detail | Owner | Contact Methodology | Date | Target | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 110101 | Road verge | N/A | N/A | 1/01/2001 | Yellow plastic sheeting used |  |
| 110102 | 1265 Great Northern Hwy | John Smith | Knocked on front door spoke to Mrs Smith | 1/01/2001 |  | Refused to allow target placement |
|  |  |  |  |  |  | Target placed in road reserve |
| 110103 | Lot 100 Great Northern Hwy | Jack Thomson | (08) 6666 6666 | 1/01/2001 | Yellow tiles used in paddock |  |
| 110104 |  |  |  |  |  |  |