Dieback and Weed Management Plan

Brand Highway Widening and Passing Lanes 65-86 SLK

October 2019

| Report Compilation & Review | Name and Position | Document Revision | Date |
| --- | --- | --- | --- |
| Author: | Emma Fitzgerald Environment Officer | Draft v1 | 10 September 2019 |
| Reviewer: | Freea Itzstein-Davey Senior Environmental Officer | Rev 0 | 16/09/2019 |
| Author: | Emma Fitzgerald Environment Officer | Rev 0 | 17/9/2019 |
| Reviewer: | Emer O’Gara DBCA | Rev 0 | 29/10/2019 |
| Author: | Emma Fitzgerald Environment Officer | Rev 1 | 30/10/2019 |

# Contents

1	PROJECT SUMMARY	4

2	INTRODUCTION	4

3	Project Description	4

3.1	Project Location	5

4	Existing STATUS	8

4.1	Dieback	8

4.1.1	Status	8

4.1.2	Locations	8

4.2	Weeds	15

4.2.1	Introduced Weeds	15

4.2.2	Locations of Weed Species	15

5	Objectives	17

5.1	Dieback	17

5.2	Weeds	17

6	Stakeholder consultation	17

7	Management and monitoring Actions	17

7.1	Dieback	17

7.2	Weeds	18

8	Reporting	20

9	Communication	20

10	Accountability	20

11	References	21

Appendix A Dieback Management Plan	22

Appendix B Dieback Clean on Entry Maps	39

Appendix C	45

Appendix D	46

Appendix E	48

# PROJECT SUMMARY

Project Title: Brand Highway Widening and Passing Lanes 65-86 SLK

Project Location:  This project occurs in the following locations on Brand Highway:

- Widening 67.5-68.63 SLK within the Shire of Gingin

- Southbound Passing Lane 68.63-71.02 SLK within the Shire of Gingin

- Widening 71.02-74 SLK within the Shire of Gingin

- Widening 77-79.84 SLK within the Shire of Dandaragan

- Northbound Passing Lane 79.84-81.66 SLK within the Shire of Dandaragan

- Widening 81.66-86 SLK within the Shire of Dandaragan

Dieback Management Objectives:

- Prevent the spread of dieback along Brand Highway.

Weed Management Objectives:

- Prevent the introduction of weeds

- Control (and if possible reduce) existing weed populations

# INTRODUCTION

Main Roads Mid-West Gascoyne Region proposes to widen sections of Brand Highway and create one northbound and one southbound passing lanes between 65 and 86 SLK. This will increase the safety of the road by providing safe overtaking opportunities and improving the functionality of the road. This dieback and weed management plan (DWMP) has been created to prescribe management actions for this project in accordance with Condition 7 of CPS 7533/2.

# Project Description

This project will involve the widening of the road, creation of one northbound and one southbound passing lane between 65 and 86 SLK within the Shire of Gingin and Dandaragan.

Widening 67.5-68.63 SLK

This project will involve the widening of the road formation between 67.5 and 68.63 SLK to a seal/pavement configuration of 11m on 11m. This includes two 3.5m lanes, two 1m sealed shoulders and two 1m unsealed shoulders. To complete this work up to 2.2ha of native vegetation will be removed.

Southbound Passing Lane 68.63-71.02 SLK

This project involves the construction of a southbound passing lane and the widening of the existing road including shoulder widening on the northbound side of the road from 68.63-71.02 SLK. This will involve the clearing of up to 2.7ha of native vegetation.

Widening 71.02-74 SLK

This project will involve the widening of the road formation between 71.02 and 74 SLK to a seal/pavement configuration of11m on 11m. This includes two 3.5m lanes, two 1m sealed shoulders and two 1m unsealed shoulders. To complete this work up to 1.8ha of native vegetation will be removed.

Widening 77-79.84 SLK

This project will involve the widening of the road formation between 77.3 and 79.84 SLK to a seal/pavement configuration of 11m on 11m. This includes two 3.5m lanes, two 1m sealed shoulders and two 1m unsealed shoulders. To complete this work up to 1.6ha of native vegetation will be removed.

Northbound Passing Lane 79.84-81.66 SLK

This project involves the construction of a northbound passing lane and the widening of the existing road including shoulder widening on the southbound side of the road from 79.84-81.1 SLK. This will involve the clearing of up to 2.1ha of native vegetation.

Widening 81.66-86 SLK

This project will involve the widening of the road formation between 81.71 and 86 SLK to a seal/pavement configuration of 11m on 11m. This includes two 3.5m lanes, two 1m sealed shoulders and two 1m unsealed shoulders. To complete this work up to 2.6ha of native vegetation will be removed.

## Project Location

The project is located on Brand Highway:

| Project Component | Start | End | Shire |
| --- | --- | --- | --- |
| Widening 67.5-68.63 SLK (CPS 7533/3-2b) | Latitude: -31.057 Longitude: 115.739 | Latitude: -31.0483 Longitude: 115.7332 | Gingin |
| Southbound Passing Lane 68.63-71.02 SLK (CPS 7533/3-2c) | Latitude: -31.0484 Longitude: 115.7332 | Latitude: -31.0281 Longitude: 115.7249 | Gingin |
| Widening 71.02-74 SLK (CPS 7533/3-2d) | Latitude: -31.0282 Longitude: 115.725 | Latitude: -31.0038 Longitude: 115.7119 | Gingin |
| Widening 77-79.84 SLK (CPS 7533/3-2e) | Latitude: -30.9788 Longitude: 115.7012 | Latitude: -30.9569 Longitude: 115.6856 | Dandaragan |
| Northbound Passing Lane 79.84-81.66 SLK (CPS 7533/3-2f) | Latitude: -30.957 Longitude: 115.6856 | Latitude: -30.9434 Longitude: 115.6749 | Dandaragan |
| Widening 81.66-86 SLK (CPS 7533/3-2g) | Latitude: -30.9434 Longitude: 115.6749 | Latitude: -30.913 Longitude: 115.6465 | Dandaragan |

Figure 1: Project Location Widening 67.5-68.63 SLK, Passing Lane 68.63-71.02 SLK and Widening 71.02-74 SLK

Figure 2: Project Location Widening 77-79.84 SLK, Passing Lane 79.84-81.66 SLK and Widening 81.66-86 SLK

# Existing STATUS

## Dieback

Phytophthora dieback (dieback) is the name used to describe the plant disease caused by a soil-borne pathogen (technically classified as a water mould not a fungus), most commonly identified as Phytophthora cinnamomi.

Dieback kills a wide range of plant species in Western Australia (WA) by attacking plant root systems and causing them to rot, thereby stopping the uptake of water and nutrients. The pathogen is spread via root to root contact, through infested soil and plant material and via surface and sub-surface water flows.

The pathogen is most rapidly spread through human activity and animals. The unintentional movement of pathogen-infested soil by vehicles, equipment and footwear from infested into uninfested areas is the most common means of dispersal of the organism. Moist soil conditions are conducive to the spread of the pathogen and the movement of the pathogen in the landscape is much faster downslope. Risks of new infestations are higher when moist soil, mud or gravel is transported from infested to uninfested areas.

The risk of soil transportation of dieback is considerably reduced in dieback susceptible areas when the movement of vehicles and machinery between infested and uninfested areas is undertaken in dry conditions. However, when activities require soil movement in wet conditions, best practice environmental management strategies should be developed and implemented to minimise soil movement and the increased risk of dieback spread or intensification. In these circumstances, specified management strategies and a project specific Dieback Management Plan (DMP) are needed to manage the potential spread of dieback and the threat to biodiversity posed by the pathogen.

### Status

A dieback assessment was undertaken by Glevan Consulting in March/April 2019 (Appendix E). Dieback is known to exist in the project area, with the majority of the project being protectable. The dieback survey identified that the majority of the project area is uninfested. There are small sections of infested areas in 81-86 SLK project area and areas infested by other Phytopthora in the 71-74 SLK and 77-79 SLK sections.

### Locations

Figure 3-8 below shows the dieback mapping of the project areas as determined during the Glevan survey.

Figure 3: Current dieback status-Widening 67.5-68.63 SLK

Figure 4: Current dieback status-Passing Lane 68.63-71.02 SLK

Figure 5: Current dieback status-Widening 71.02-74 SLK

Figure 6: Current dieback status-Widening 77-79.84 SLK

Figure 7: Current dieback status-Passing Lane 79.84-81.66 SLK

Figure 8: Current dieback status-Widening 81.66-86 SLK

## Weeds

Weeds are not native to an area and are generally opportunistic plant species.  Once weeds are introduced to an area they effectively compete for resources.  The presence of weeds in an area can result in numerous environmental impacts including resource competition, the prevention of seedling recruitment of native plant species, changes to soil nutrients and fire regimes and alteration of geomorphological and hydrological cycles.

### Introduced Weeds

One weed species was recorded within the Project Area during the GHD 2016 survey (refer to Table 1).  Echium plantagineum (Paterson’s Curse) is categorised as Declared Pests under the Biosecurity and Agriculture Management Act 2007.

One Paterson’s Curse individual was observed at one location within the project area.  It is an erect, growing up to 1.5m tall high, commonly 30-60cm and the flowers are purple. This species is widespread throughout the south-west of WA and the eastern Goldfields.

| Table 1: Weed species in project area |  |  |  |
| --- | --- | --- | --- |
| Species Name | Common Name | Latitude | Longitude |
| Echium plantagineum | Paterson’s Curse | -31.0369 | 115.7294 |

### Locations of Weed Species

Figure 9 below shows the location of Paterson’s Curse with the project area.

Figure 9. Weed location in project area

# Objectives

## Dieback

The objective of the DWMP is to prescribe management actions to avoid, mitigate and manage the potential introduction of dieback within the project area. As dieback is usually spread during the movement of soil and vegetative material, management actions will focus on these aspects.

## Weeds

Management objectives with respect to mitigating potential environmental impacts have been developed, consistent with statutory requirements and commitments, to effectively manage weeds as follows:

- Prevent the introduction of weeds

- Control (and if possible reduce) existing weed populations

For each objective, management actions have been developed to ensure the impacts from construction operations are managed, and that appropriate monitoring is undertaken to support the successful implementation of the management actions.

# Stakeholder consultation

The DWMP and associated management actions have been prepared in line with the DBCA’s Phytophthora Dieback Management Manual. This plan was reviewed by Emer O’Gara as it occurs adjacent to Department of Biodiversity, Conservation and Attractions (DBCA) land. The plan was also approved by Department of Water and Environmental Regulation (DWER).

# Management and monitoring Actions

## Dieback

The Dieback Management Plan is attached as Appendix A.

## Weeds

| The contractor shall implement the weed management actions outlined in Table 2 below. Table 2: Weed management actions |  |  |  |
| --- | --- | --- | --- |
| Management Action | Monitoring/Maintenance Program | Responsible Person | Completion Timeframe |
| Preconstruction |  |  |  |
| Preconstruction survey shall be undertaken to determine extent of weeds in the project area | Pre-construction | Consultant | Completed |
| At the pre-start meeting (or equivalent) – provide clear maps indicating the location of weeds (limited to the project area) | Pre-construction / Construction | Environmental Management Representative | Prior to clearing commencing |
| Have on site a copy of the Weed Management Plan | Pre-construction / Construction | Environmental Management Representative | Prior to clearing commencing |
| Where known locations of weeds will be impacted by construction activities, undertake weed control through removal and burial at approved location | Pre-construction / Construction | Environmental Management Representative | Prior to clearing commencing |
| Construction |  |  |  |
| Ensure all plant and equipment are clean, inspected and certified weed free prior to entering the project area | Construction | Environmental Management Representative | Completion of construction |
| Restrict movement of machines and other vehicles to the limits of the project area | Construction | Environmental Management Representative | Completion of construction |
| Any other weeds or plants germinating from stockpiled topsoil are to be controlled with herbicide | Construction | Environmental Management Representative | Completion of construction |
| Any stockpiles with Paterson’s Curse germinating are to be disposed of as weedy topsoil | Construction | Environmental Management Representative | Completion of construction |

| Management Action | Monitoring/Maintenance Program | Responsible Person | Completion Timeframe |
| --- | --- | --- | --- |
| Monitoring |  |  |  |
| Weed monitoring to be undertaken post construction | Post-construction | Site Supervisor | Expiry of defects period |
| Where weed infestation is evident, herbicide application shall be undertaken to ensure no establishment of declared weed species | Construction / Post-construction | Site Supervisor | Expiry of defects period |
| Contractor shall monitor site where weeds are buried (in approved location) to ensure no establishment of declared weed species | Construction / Post-construction | Site Supervisor | Expiry of defects period |

# Reporting

The Contract Project Manager will maintain the following records, which will be reported annually to DWER as required under the conditions of CPS 7533/2.

- Copy of this DWMP, DBCA and DWER approval letters

- Copy of the dieback survey report; and

- actions taken in implementing this DWMP.

# Communication

Main Roads employees, contractors and agents who are involved in the project activities and who have the potential to introduce and spread dieback or weeds into uninfested areas, should read and comprehend this DWMP. The dieback management map will be available on site in the site office and supervisor vehicles. The dieback management sectors will also be demarcated on site with orange flagging tape and the CoE points will be identified. Information regarding dieback and weed management will be communicated to all project employees as follows:

Table 3	Communication Program

| Method | Frequency | Participants | Reference | Record |
| --- | --- | --- | --- | --- |
| Induction | Prior to commencement of work | All personnel and sub-contractors | DWMP, Main Roads’ Environmental Policy Statement and CEMP | Induction meeting |
| Training | Prior to commencement of work | All personnel and sub-contractors | DWMP and toolbox | Records of training |
| Pre-start Meetings | Daily | Project Personnel | DWMP, Contractor Environment & Safety Plan | Minutes of meeting |
| Superintendent meeting | Fortnightly | Main Roads’ Project Manager and Contract Project Manager | DWMP | Minutes of meeting |

# Accountability

The Project Manager and Contractor are responsible for ensuring the dieback and weed management actions, monitoring program and timeframes for completion are undertaken.

# References

Astron (2016) Brand Highway, Regans Ford Biological Survey. Unpublished report to Main Roads Western Australia.

GHD (2016) Brand Highway Passing Lanes Biological Assessment. Unpublished report to Main Roads Western Australia.

Glevan (2019) Brand Cockram Intersection and SLK 67-86 SLK. Unpublished report to Main Roads Western Australia.

Department of Agriculture and Food (2019) Western Australian Organism List (WAOL).

https://www.agric.wa.gov.au/bam/western-australian-organism-list-waol Accessed 9 September 2019.

# Appendix A

# Dieback Management Plan

Phytophthora Dieback Management Plan

PART A: COVER PAGE

This form is to be used for assessing the dieback risks associated with planned disturbance activities on lands managed by the Department of Biodiversity, Conservation and Attractions, for documenting the risk reduction tactics and monitoring implementation of the plan.

The form has three parts, and at least two parts will need to be completed for each disturbance activity. Please indicate in the table below which parts have been completed for this disturbance activity.

| Part | Purpose | Requirement | Tick those completed |
| --- | --- | --- | --- |
| Part A | Cover Page (instructions, details about the activity) | Must be completed (if activity requires more than basic dieback management as per section 1.4 of manual) |  |
| Part B | Risk Assessment | To be conducted for all planned disturbance activities in the vulnerable zone on lands managed by the department, except: Where risk is already known to be ‘High’ a proponent may choose to go directly to Part C. |  |
| Part C | Management Actions | Where the risk has been assessed as ‘High’ or ‘Moderate’ a management map is to be included. Part C is not required where the risk has been assessed to be ‘Low’. In this case, the proponent can proceed using basic dieback management. |  |

| Dieback Management Plan No. District allocated - to be recorded on all parts of plan submitted |  |
| --- | --- |

Details of the disturbance activity

| Region/District of activity: | Swan Coastal and Moora Districts | Date of activity: (give date range if a prolonged activity) | Clearing programmed mid October till November 2019. Project expected to be completed in April 2020. |
| --- | --- | --- | --- |
| Location of site of activity: (Forest Block, Reserve or coordinates) | Refer to Section 3.1 above |  |  |
| Vegetation type/complex: | Widening 67.5-68.63 SLK - Highly modified-Includes areas that have been predominantly cleared but which still contain either scattered natives or introduced species. These include bulldozed firebreaks that contain seed and which may support occasional native species - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Banksia Woodland-Banksia attenuata and B. menziesii low woodland over Adenanthos cygnorum subsp. cygnorum and Eremaea pauciflora var. pauciflora open shrubland over Stirlingia latifolia low open shrubland over Mesomelaena pseudostygia very open sedgeland. Southbound Passing Lane 68.63-71.02 SLK - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Highly modified-Includes areas that have been predominantly cleared but which still contain either scattered natives or introduced species. These include bulldozed firebreaks that contain seed and which may support occasional native species Widening 71.02-74 SLK - Melaleuca preissiana Woodland-Woodland of Melaleuca preissiana over tall shrubland of M. incana subsp. incana, Hypocalymma angustifolium, Gastrolobium obovatum over closed sedgeland of Gahnia trifida, Juncus kraussii subsp., australiensis, Schoenus caesipitius over sparse herbland of Laxmannia ramosa subsp. ramose, Drosera erythrorhiza, Isotropis cuneifolia subsp. cuneifolia - Highly modified-Includes areas that have been predominantly cleared but which still contain either scattered natives or introduced species. These include bulldozed firebreaks that contain seed and which may support occasional native species - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Banksia Woodland-Banksia attenuata and Banksia menziesii low woodland over Adenanthos cygnorum subsp. cygnorum and Eremaea pauciflora var. pauciflora open shrubland over Stirlingia latifolia low open shrubland over Mesomelaena pseudostygia very open sedgeland. - Pl04-Eucalyptus todtiana, Banksia attenuata and Banksia menziesii low open woodland over Xanthorrhoea preissii open shrubland over Hibbertia crassifolia, Eremaea pauciflora var. pauciflora and Allocasuarina humilis low shrubland over Mesomelaena pseudostygia and Tetraria octandra very open sedgeland. - Pl05-Corymbia calophylla low open forest over Xanthorrhoea preissii and Hakea trifurcata open shrubland over Bossiaea eriocarpa and Jacksonia sternbergiana low shrubland over Mesomelaena pseudostygia and Caustis dioica open sedgeland. Widening 77-79.84 SLK - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Banksia Woodland on White Sand-Low Open Woodland dominated by Banksia attenuata, B. menziesii with Eucalyptus todtiana over Scattered Shrubs of B. attenuata, Jacksonia floribunda over Low Shrubland to Open Shrubland of Melaleuca urceolaris and Eremaea pauciflora over Sedgeland of Desmocladus subterranea, Xanthorrhoea priessii over herbs Trachymene pilosa, Podotheca angustifolia, Burchardia congesta, Pterostylis dilatata on White Sand. - Cleared/ Degraded-Includes existing clearing road reserve, gravel pits, tracks and firebreaks. Often comprises Scattered Shrubs over Scattered Bunch introduced grass and Scattered Herbs. - Geomorphic Wetland-Trees of Corymbia calophylla, Melaleuca priessiana over Shrubs of M. priessiana, M. incana, Hypocalymma angustifolium over Sedgeland of Lepidosperma squamatum, *Cyperus congestus - Marri Banksia Woodland-Woodland of Corymbia calophylla over Low Woodland of Banksia attenuata, Banksia prionotes over High Open Shrubland of Allocasuarina humilis, Nuytsia floribunda over Scattered Shrubs of Jacksonia horrida, Hibbertia hypericoides over Low Shrubland of Calothamnus sanguineus, Acacia pulchella var. glaberrima, Banksia nivea over Open Sedgeland of Caustis dioica, Mesomelaena pseudostygia over Scattered Herbs of Cassytha flava, Haemodorum brevisepalum. - Heath and Emergents-Scattered Low Trees of Banksia attenuata, Eucalyptus todtiana over Scattered Heath of Leucopogon oldfieldii, Astroloma xerophyllum, Hibbertia aurea, Hakea obliqua, H. ruscifolia, Melaleuca trichophylla over Low Shrubland of Conostylis, Calothamnus sanguineus over Very Open Sedgeland of Mesomelaena pseudostygia, Desmocladus subterranea, Lyginia barbarta over Scattered Herbs of Drosera spp. and Burchardia congesta. Northbound Passing Lane 79.84-81.66 SLK - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Highly modified-Includes areas that have been predominantly cleared but which still contain either scattered natives or introduced species. These include bulldozed firebreaks that contain seed and which may support occasional native species - Tall Mixed Shrubland-Tall shrubland of Xanthorrhoea preissii, Hakea trifurcata and Jacksonia sternbergiana over shrubland of Hibbertia hypericoides, Petrophile macrostachya and Calothamnus quadrifidus subsp. quadrifidus and grassland of Austrostipa elegantissima, *Ehrharta longiflora, Neurachne alopecuroides over open herbland of Drosera erythrorhiza, Hypochaeris glabra, *Ursinia anthemoides Widening 81.66-86 SLK - Banksia attenuata and B. menziesii woodland-Woodland of Banksia menziesii and B. attenuata over sparse tall shrubland of Adenanthos cygnorum, Jacksonia floribunda and Xanthorrhoea preissii over low shrubland of Bossiaea eriocarpa, Eremaea pauciflora, Stirlingia latifolia and open sedgeland of Lyginia barbata, Alexgeorgea nitens and Mesomelaena pseudostygia over sparse herbland of Haemodorum sp., Drosera erythrorhiza and Dampiera linearis - Highly modified-Includes areas that have been predominantly cleared but which still contain either scattered natives or introduced species. These include bulldozed firebreaks that contain seed and which may support occasional native species - Banksia Woodland-Banksia attenuata and Banksia menziesii low woodland over Adenanthos cygnorum subsp. cygnorum and Eremaea pauciflora var. pauciflora open shrubland over Stirlingia latifolia low open shrubland over Mesomelaena pseudostygia very open sedgeland. - Pl02-Grevillea eriostachya and Allocasuarina humilis tall open shrubland over Xanthorrhoea preissii and Eremaea pauciflora var. pauciflora open shrubland over Austrostipa elegantissima and Amphipogon turbinatus very open tussock grassland over Mesomelaena pseudostygia very open sedgeland. - Pl03-Calothamnus quadrifidus subsp. quadrifidus, Allocasuarina humilis and Jacksonia floribunda tall shrubland over Eremaea pauciflora var. pauciflora and Xanthorrhoea preissii shrubland over Hibbertia crassifolia low open shrubland over Tetraria octandra, Mesomelaena pseudostygia open sedgeland. - Pl04-Eucalyptus todtiana, Banksia attenuata and Banksia menziesii low open woodland over Xanthorrhoea preissii open shrubland over Hibbertia crassifolia, Eremaea pauciflora var. pauciflora and Allocasuarina humilis low shrubland over Mesomelaena pseudostygia and Tetraria octandra very open sedgeland. - W01-Banksia prionotes and Melaleuca rhaphiophylla tall shrubland over Acacia saligna subsp. saligna open shrubland over Juncus kraussii subsp. australiensis low open shrubland over *Ehrharta calycina very open tussock grassland. (GHD 2016 and Astron 2016) |  |  |
| Description of the activity: (timber harvesting, road upgrade etc.) | Refer to Section 3 above |  |  |
| Proponent of the activity: (DBCA, FPC, MRWA, Water Corp. etc.) | Main Roads Western Australia |  |  |
| Departmental objective for dieback management: | To minimise the potential for the introduction or spread of dieback associated with planned disturbance activities. |  |  |

PART B: RISK ASSESSMENT

| Dieback Management Plan No. District allocated - to be recorded on all parts of plan submitted |  |
| --- | --- |

Step 1:	Forecast the MOISTURE conditions during the activity

Higher moisture during the activity will increase the likelihood that soil will stick to vehicles, equipment and/or footwear.  Tick the box adjacent to the moisture conditions that are forecast for the period of the activity.  If the activity will continue for an extended period, planning should consider the highest possible risk (wettest) conditions that may occur. If during a disturbance activity soil moisture becomes higher than anticipated, the DMP should be reviewed to confirm that all management actions remain appropriate (assuming the risk rating of the activity increased with the additional moisture).

| Dry soil | where dust forms when exposed soil is disturbed |  |
| --- | --- | --- |
| Moist soil | where soil is damp but does not stick to tyres, equipment and/or footwear |  |
| Wet soil | where soil and moisture combine so that soil sticks to tyres, equipment and/or footwear |  |

Step 2:	Determine the LIKELIHOOD of introducing or spreading dieback

Circle the description in each column that best describes the activity. An activity may fit between descriptions, in which case write a description into the appropriate blank cell.

The overall likelihood rating is determined by the factor with the highest rating.

| Disturbance type (e.g. action) | Introduction of raw material | Access | Complexity of activity | Extent of activity | Duration of activity | Drainage | Unmanaged access | Likelihood rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy earth moving, tracked vehicles | Infested or unknown raw material | Access crosses water (irrespective of frequency) |  |  | Activity area disturbed & map expired so impossible to revalidate boundaries |  | Increased public access in area of high public use | Almost certain |
| Soil disturbance over a distance |  | Activity requires frequent access to site | Highly complex | Vehicle traverses several mini-catchments | Activity extends over several wet seasons | Surface water increased |  | Likely |
| Soil disturbance at single points | Crushed rock with no organic fraction |  | Complex |  | Activity occurs during a single wet season |  | Increased public access, but access restricted and/or site remote | Possible |
| Rubber tyred vehicle, bicycle | ‘High confidence’ uninfested raw material | Activity requires infrequent access to site |  | Single mini-catchment | Entry in short timeframe under dry conditions | Minimal increase in surface water |  | Unlikely |
| Human, animal traffic |  |  | Not complex | Point or human traffic | Single entry in short timeframe under dry conditions |  | Activity does not alter frequency of access to site | Very unlikely |

Step 3:	Determine the CONSEQUENCE of introducing or spreading dieback

Determine the potential CONSEQUENCE that introducing or spreading dieback may cause by going through the table below systematically and circling the description in each column that best estimates the consequence.

The overall consequence rating is determined by the factor with the highest rating.

| Area put at risk | Predicted impact | Biodiversity and sensitive areas at risk | Consequence rating |
| --- | --- | --- | --- |
| Potential to infest all protectable areas in activity landscape unit* | Predicted high or very high impact: (majority of species at the activity area are susceptible and/or introducing dieback will result in extinction of species or populations) or Wet areas which contain any Banksia species or jarrah | >1 threatened/priority plant or animal species, critical habitat, TEC and/or Ramsar wetlands that is susceptible to dieback and/or - Old-growth jarrah forest - Project Dieback PPA (uninfested high value hotspot) | Severe |
| Potential to infest more than 5% of any protectable area or 4 ha’s (whichever is greater – assessor may set a lower minimum protectable area where appropriate) | Predicted high impact: (many susceptible species and/or introducing the pathogen will result in loss of populations or localised extinction of species) or Where predicted impact cannot be determined, jarrah forest on upland areas | At least one threatened/priority plant or animal species, critical habitat, TEC and/or Ramsar wetlands that is susceptible to dieback and/or - Project Dieback PPA (uninfested high value landscapes) - Sensitive neighbouring property | Significant |
|  | Predicted moderate impact: (moderate numbers of susceptible species and/or introducing the pathogen will result in a reduction in species/populations) |  | Intermediate |
|  | Predicted low impact (low numbers of susceptible species) | Fauna Habitat Zones | Minor |
| No protectable areas estimated within any related landscape unit | No susceptible species and/or the activity area is in the ‘excluded’ category. Introducing dieback will have no impact discernible outside natural variation | No threatened/priority plant or animal species; critical habitat; TEC; Ramsar wetlands that are susceptible to dieback and/or; PPAs | Insignificant |

*Landscape unit is an area bounded by features such as creeks, ridges, saddles, open roads and/or freehold land

Step 4:	Determine the overall dieback RISK rating

- Refer to the table below that corresponds to the soil MOISTURE conditions (Step 1)

- Circle where the LIKELIHOOD rating (Step 2) intersects the CONSEQUENCE rating (Step 3)

This is the overall dieback RISK rating for the activity.

| DRY SOIL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CONSEQUENCE |  |  |  |  |
| LIKELIHOOD | Insignificant | Minor | Intermediate | Significant | Severe |
| Almost certain | Low | Moderate | High | High | High |
| Likely | Low | Moderate | Moderate | High | High |
| Possible | Low | Low | Moderate | Moderate | High |
| Unlikely | Low | Low | Low | Moderate | High |
| Very unlikely | Low | Low | Low | Moderate | High |
| MOIST SOIL |  |  |  |  |  |
|  | CONSEQUENCE |  |  |  |  |
| LIKELIHOOD | Insignificant | Minor | Intermediate | Significant | Severe |
| Almost certain | Low | High | High | High | High |
| Likely | Low | Moderate | High | High | High |
| Possible | Low | Moderate | Moderate | High | High |
| Unlikely | Low | Low | Moderate | Moderate | High |
| Very unlikely | Low | Low | Low | Moderate | High |
| WET SOIL |  |  |  |  |  |
|  | CONSEQUENCE |  |  |  |  |
| LIKELIHOOD | Insignificant | Minor | Intermediate | Significant | Severe |
| Almost certain | Low | High | High | High | High |
| Likely | Low | High | High | High | High |
| Possible | Low | Moderate | High | High | High |
| Unlikely | Low | Moderate | Moderate | High | High |
| Very unlikely | Low | Low | Moderate | Moderate | High |

Step 5:	Can the risk be reduced by altering the activity or conditions?

If the risk rating is ‘high’ it is recommended that the activity be re-assessed to determine if the risk can be reduced by altering some of the parameters of the activity. Is the activity absolutely necessary?  Changing season or moisture conditions under which the activity will be done could have a significant impact on the risk, as could changing the site of the activity, or the equipment used.

Step 6:	Determine requirements based on RISK rating

Tick the box adjacent to the RISK rating of the activity as determined by the risk table.

| High | - Complete Part C based on valid and comprehensive dieback interpretation with Regional Manager (or delegate) approval before implementation, and sign-off after close-out - Green Card training1 for all proponents and contractors involved in activity |  |
| --- | --- | --- |
| Moderate | - Complete Part C based on valid and comprehensive dieback interpretation OR conditional dieback occurrence information with Regional Manager (or delegate) approval before implementation, and sign-off after close-out - Green Card training1 for proponent and contractors involved in activity |  |
| Low | - Part C not required. Activity can proceed using basic dieback management - Green Card training1 for all proponents and contractors involved in activity |  |

1 Green Card training is mandatory for nominated departmental staff

Step 7:	Risk Assessment sign-off

|  | Full Name | Position | Signature | Date |
| --- | --- | --- | --- | --- |
| Risk Assessment conducted by: | Emma Fitzgerald | Environment Officer |  | 9/9/2019 |

Additional comments or conditions:

PART C: MANAGEMENT ACTIONS

| Dieback Management Plan No. District allocated - to be recorded on all parts of plan submitted |  |
| --- | --- |

Step 1:  Dieback occurrence information & map (supervising officer/proponent)

| Valid and comprehensive interpretation |  | or | Conditional information |  |  |
| --- | --- | --- | --- | --- | --- |
| Report no. &/or name | Appendix E |  | Source |  |  |
| Map type(s) &/or code | Appendix E and Figures 3-8 above |  | Management map attached with all approved management points clearly marked? |  |  |

Step 2:	DMP meeting (supervising officer/proponent)

| Date: |  | Convened by: |  |
| --- | --- | --- | --- |
| Attended by: |  |  |  |

Step 3:	Risk management tactics (supervising officer/proponent)

| Tactic no. | TACTICS TO BE DEPLOYED |  |  |  |  | To be implemented (= required, X = not required) | Implemented (initialled when complete) | Checked (initialled when checked) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOISTURE CONDITIONS |  |  |  |  |  |  |  |  |
| 1 | Moisture conditions (tick permitted) The works will be completed between November and April so will be conducted in dry conditions unless unseasonal rains. Works only be conducted next to the conservation area in dry conditions |  | dry_____ | moist_____ | wet_____ | X |  |  |
| 2 | Operations to cease in the event of rain |  |  |  |  | X |  |  |
| PROTECTABLE AREAS |  |  |  |  |  |  |  |  |
| 3 | Protectable area boundaries which are clearly marked in field and on map or |  |  |  |  | X |  |  |
| 4 | Disease categories within protectable areas which are clearly marked in field and on map |  |  |  |  |  |  |  |
| HYGIENE |  |  |  |  |  |  |  |  |
| 5 | Management points installed and functional by following date ___1__/10_____/2019______ |  |  |  |  |  |  |  |
| 6 | ___10__ Clean on Entry (COE) points established and numbered on the map |  |  |  |  |  |  |  |
| 7 | _____ COE gates installed and indicated on map against COE no. |  |  |  |  | X |  |  |
| 8 | _____ turnarounds for COE points, numbered and marked on map |  |  |  |  | X |  |  |
| 9 | COE points closed and rehabilitated within ___6__ weeks of completion |  |  |  |  |  |  |  |
| 10 | Cleandown points established and indicated on map |  |  |  |  |  |  |  |
| 11 | Machines and vehicles with portable hygiene kits |  |  |  |  |  |  |  |
| 12 | Records kept (circle relevant): COE cleandown |  |  |  |  |  |  |  |
| COMMUNICATION |  |  |  |  |  |  |  |  |
| 13 | Staff/contractors with Green Card training |  |  |  |  | X |  |  |
| 14 | DMP briefings (circle relevant): at commencement weekly daily other |  |  |  |  |  |  |  |
| 15 | Staff and contractors in field have copy of the dieback management map |  |  |  |  |  |  |  |
| 16 | Total no of signs ___1__ abbreviate the type _____ _____ _____ _____ Signs to be installed within dieback infested parking bay, bay will also be sealed. |  |  |  |  |  |  |  |
| DISTURBANCE |  |  |  |  |  |  |  |  |
| 17 | Machinery type: graders, loaders, dozers (maybe), bobcat, side tippers, excavators |  |  |  |  |  |  |  |
| 18 | ‘No soil movement’ roads and areas identified on map |  |  |  |  | X |  |  |
| 19 | Records kept for access on ‘No soil movement’ roads |  |  |  |  | X |  |  |
| RAW MATERIALS |  |  |  |  |  |  |  |  |
| 20 | Type: Gravel |  | Supplier/Source: Catalano’s pit |  |  |  |  |  |
| 21 | Status (include evidence): Unknown, screened gravel only no vegetative matter. Gravel source has been used previously and no dieback is present at the sites where it was used. |  |  |  |  |  |  |  |
| ACCESS |  |  |  |  |  |  |  |  |
| 22 | Access route planned to place least amount of protectable area downslope at risk, and shown on map |  |  |  |  | X |  |  |
| 23 | Road maintenance uses tactics to mitigate harm to protectable areas: | use interpreted boundaries |  |  |  | X |  |  |
| 24 |  | push soil downslope only |  |  |  | X |  |  |
| 25 |  | clean bucket, shovel, auger after digging culverts/holes |  |  |  | X |  |  |
| 26 |  | use uninfested/low risk material to patch road |  |  |  | X |  |  |
| 27 | _____ roads to be closed, each road closure is numbered and marked on map |  |  |  |  | X |  |  |
| 28 | Each road closure has been constructed to effectively control access |  |  |  |  | X |  |  |
| 29 | Roads closed/rehabilitated within _____ weeks of end of activity |  |  |  |  | X |  |  |
| 30 | Road construction uses tactics to mitigate harm to protectable areas: | located in infested/unprotectable categories when possible |  |  |  | X |  |  |
| 31 |  | low in profile |  |  |  | X |  |  |
| 32 |  | high crown for better drainage |  |  |  | X |  |  |
| 33 |  | deep roadside drains & coarse material to minimise erosion |  |  |  | X |  |  |
| 34 |  | Mitre/offshoot drain preferentially located toward the base of the slope |  |  |  | X |  |  |
| 35 | ‘Green-bridge’ implemented |  |  |  |  | X |  |  |
| 36 | Activity to be undertaken using split-phase |  |  |  |  | X |  |  |

| Tactic no. | TACTICS TO BE DEPLOYED |  |  |  | To be implemented (= required, X = not required) | Implemented (initialled when complete) | Checked (initialled when checked) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EXTENT |  |  |  |  |  |  |  |
| 37 | Divided into two management units, one unit for dry/moist and one unit for wet |  |  |  | X |  |  |
| 38 | Select factors to be used to split dry soil and wet soil management units |  | 1 | Protectability | X |  |  |
| 39 |  |  | 2 | Presence of biodiversity values | X |  |  |
| 40 |  |  | 3 | Predicted impact | X |  |  |
| 41 |  |  | 4 | Potential for spread | X |  |  |
| 42 |  |  | 5 | Machine/vehicle floatation | X |  |  |
| 43 |  |  | 6 | Access | X |  |  |
| 44 |  |  | 7 | Unmanaged access | X |  |  |
| 45 |  |  | 8 | Duration | X |  |  |
| 46 | Exclude use of moist conditions in the dry soil unit |  |  |  | X |  |  |
| 47 | Operate to mini-catchments |  |  |  | X |  |  |
| DURATION |  |  |  |  |  |  |  |
| 48 | Duration of activity >1 year, engage Interpreter to extend buffers, even if vegetation is disturbed |  |  |  |  |  |  |
| DRAINAGE |  |  |  |  |  |  |  |
| 49 | Drainage directed away from protectable areas |  |  |  | X* |  |  |
| 50 | Imported water | Source: To be sourced by contractor |  |  |  |  |  |
| 51 |  | Disinfectant: |  |  | X |  |  |
| ADDITIONAL CONDITIONS |  |  |  |  |  |  |  |
|  | See table 4 below |  |  |  |  |  |  |
|  | *The majority of the work site will have a drain that will be sufficient to capture any runoff, however there is a section of 350m adjacent to the national park where this is not the case. Three years worth of monitoring will occur (at year 1, 3 and 5) and if any issues arise a sump or other drainage solutions will be considered. |  |  |  |  |  |  |

Step 4:	DMP approval (Regional Manager (or delegate))

I, the undersigned, have reviewed the Risk Assessment and approved the DMP:

| Full Name | Position | Signature | Date |
| --- | --- | --- | --- |

Step 5:	DMP close-out (supervising officer/proponent)

| All tactics identified in the DMP were implemented as approved? | Yes |  | No |  |  |
| --- | --- | --- | --- | --- | --- |

Comment:

| Full Name | Position | Signature | Date |
| --- | --- | --- | --- |

Step 6:	DMP sign-off (Regional Manager (or delegate))

I, the undersigned, am satisfied that the DMP has been implemented and closed-out as approved:

| Full Name | Position | Signature | Date |
| --- | --- | --- | --- |

Step 7:	Document management checklist

| Records ticked below are filed in the following location: |  |
| --- | --- |

|  | Dieback occurrence information (Interpretation report and map) have been forwarded to FMB at femweb@dbca.wa.gov.au |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dieback Management Map |  |  |  |  |  |
|  | Dieback Management Plan Form |  |  |  |  |  |
|  |  | Cover Page |  | Part A |  | Part B |
|  | COE and cleandown records |  |  |  |  |  |

Table 4: Dieback Management Actions

| Brand Highway SLK 67-86 - Phytophthora Dieback Management Actions and Monitoring Programme |  |  |  |  |
| --- | --- | --- | --- | --- |
| Project Component | Management Action | Monitoring Programme | Responsible Person | Completion Timeframe |
| Pre-Construction Dieback Survey and Assessment |  |  |  |  |
| Survey and Assess Dieback Status | Undertake a Dieback survey of the project area using a Phytophthora Dieback Interpreter registered with DBCA, prior to the commencement of project activities. | As part of pre-construction environmental approvals. | Environment Officer | Completed |
|  | Identify and map the Dieback status of the project area (and surrounds) from the Dieback survey. |  |  |  |
| Identify Dieback Occurrence Areas | Dieback interpretation boundaries to be marked on site as per DBCA standards | As part of pre-construction environmental approvals. | Environment Officer | Completed |
|  | This tape must be renewed and remarked at 12 monthly intervals; re-assessment due March 2020. | Will require potential re-check during project depending upon project timeline |  | Potential recheck depending upon project timeline |
| Project Specific Aspects and Clearing Permit Approval |  |  |  |  |
| Dieback Controls | All vehicles, machinery and tools shall arrive at site clean of soil, mud or vegetative material, will be inspected for compliance and sterilised prior to work if necessary (full hygiene clean down) | Daily visual inspections and vehicle inspection log kept at project site. | Project Manager / Site Supervisor | During project works |
|  | No dieback affected-soil or mulch, fill to be imported to the Project Area at any time | During construction planning; appropriate records of source of any such material and associated proof of dieback-status brought into the Project site to be retained | Site Supervisor / Contractor | Throughout Project Works |
|  | Movements of machines and other vehicles to be restricted to the areas of clearing for the project | During construction planning | Site Supervisor / Contractor | Throughout Project Works |
|  | Soil movement within a hygiene category permissible. No soil movement or movement of vegetation material to occur between hygiene categories | During construction planning | Site Supervisor / Contractor | Throughout Project Works |
| Training | Staff on site must be trained in dieback washdown procedures as well as vehicle hygiene checks | Evidence of training and site inductions to be provided to Main Roads including names of participating staff and date | Site Supervisor / Contractor | Prior to commencement of work at site |
|  | Training (induction/toolbox) undertaken for site personnel including sub-contractors regarding dieback and the importance of preventing the spread of the pathogen |  |  | Throughout Project Works |
| Dieback Management and Hygiene Plan |  |  |  |  |
| Dieback Controls Required - Non Dry Soil Conditions | Clearing will be undertaken in non-dry conditions only if unavoidable to reduce the risk of spread of the pathogen. | Project timing | Project Manager / Site Supervisor | Throughout Project Works |
|  | Vehicles, machinery and tools should be inspected to be free of soil, mud or plant matter as well as other contaminants before work is commenced and sterilized if necessary (complete hygiene washdown) | Daily visual inspections and vehicle inspection log kept at project site. |  |  |
|  | If material is clinging to equipment or machinery in wet conditions, washdown will be required to remove material prior to commencing work and when moving between hygiene categories | Maintain weekly tool box meetings and minutes of each meeting to notify of dieback infested / uninfested / uninterpretable areas and breaches of the DMP. |  |  |
|  | Works only be conducted next to the conservation area in dry conditions | Project timing |  |  |
| Standard Record Keeping Management |  |  |  |  |
| Dieback Management Plan | The DMP to be submitted to CEO (DWER) for approval prior to associated clearing works; if modifications are required, these modifications are to be made and approved by CEO prior to clearing activities | Part of pre-construction approvals | Environmental Officer | Prior to commencement of work at site |
| GIS Data Record Keeping | Dieback interpretation mapping GIS data logged as per current Dieback Data Specification | To be included in data provision during reporting | Environmental Officer | Completed |
| Record Keeping | Ensure standard record keeping requirements are completed. | During construction audits by Environmental Officer | Environmental Officer; Project Manager | Within 2 months of completion |
|  | Copies of daily hygiene logs, training records and imported material dieback status to be provided to Environment Officer | Post-construction record maintenance to occur within 2 months of completion of the project activities. |  |  |

# Appendix B

# Dieback Clean on Entry Maps

Figure 10: Clean on Entry points-Widening 67.5-68.63 SLK

Figure 11: Clean on Entry points- Passing Lane 68.63-71.02 SLK

4

3

2

1

Figure 12: Clean on Entry points- Widening 71.02-74 SLK

6

5

Figure 13: Clean on Entry points- Widening 77-79.84 SLK

Figure 14: Clean on Entry points- Passing Lane 79.84-81.66 SLK

10

9

8

7

Figure 15: Clean on Entry points- Widening 81.66-86 SLK

# Appendix C

Dieback Classification Table

Three categories are shown on a P. cinnamomi Occurrence Map; Infested (dieback occurs), Uninfested (dieback-free) or Uninterpretable (dieback unknown).

Table 5	Classification of P. cinnamomi Occurrence (CALM, 2003).

| Unmappable Areas that are sufficiently disturbed so that P. cinnamomi occurrence mapping is not possible at the time of inspection. |  |  |
| --- | --- | --- |
| Mappable Natural undisturbed vegetation. P. cinnamomi occurrence mapping is possible. Three categories may result. | Infested (dieback occurs) | Areas that have been determined to have plant disease symptoms consistent with the presence of the pathogen P. cinnamomi by a qualified disease interpreter. |
|  | Uninfested (dieback-free, also known as protectable areas) | Areas that have been determined to be free of the pathogen P. cinnamomi by a qualified Disease Interpreter (all susceptible indicator plant species are healthy; no plant disease symptoms normally attributed to P. cinnamomi are evident). |
|  | Uninterpretable (dieback unknown) | Areas situated in locations which receive > 600mm per annum rainfall or are water gaining sites (eg. granite outcrops, impeded drainage or engineering works which aggregate rainfall) where indicator plants are absent or too few to determine the presence or absence of disease caused by P. cinnamomi. |

*Source: Adapted from (CALM, 2003) Phytophthora cinnamomi and Disease Caused By It, Volume I – Management Guidelines.

# Appendix D

Dieback Contingency Actions

| DIEBACK CONTINGENCY ACTIONS |  |  |  |
| --- | --- | --- | --- |
| Project Component | Trigger | Action | Responsible Person |
| Vegetation Clearing / Revegetation Activities | Susceptible native vegetation that was previously healthy shows signs of dieback including uniform foliage chlorosis and, progressive death of susceptible native species. | - Engage a registered Phytophthora Dieback Interpreter to confirm the presence, determine the extent and identify the likely cause of Dieback spread. - Implement dieback control measures to contain new areas of dieback infestation in accordance with the DMP. - Engage a dieback specialist to monitor the levels of dieback infestation annually for three years. - Implement fencing and access around dieback uninfested areas. | Project Manager |
| Hygiene | Dieback has been introduced into an additional area(s) that were previously uninfested. | - Investigate the cause of the incursion of dieback into previously uninfested areas and review construction procedures. - Engage a registered Phytophthora Dieback Interpreter to undertake sampling to determine whether dieback infestation has occurred. - Implement control measures to contain new areas of infestation in accordance with the DMP. - Engage a dieback specialist to monitor levels of dieback infestation on an annual basis for a period of 3 years. | Project Manager |
|  | Project records indicate unauthorised vehicle or machinery access into excluded areas or breach of vehicle hygiene procedure. | - Investigate the cause of the incursion of dieback into previously uninfested areas and review construction procedures. - Engage a registered Phytophthora Dieback Interpreter to undertake sampling to determine whether dieback infestation has occurred. - Implement control measures to contain additional areas of dieback infestation. | Project Manager |
| Surface Water | Evidence of surface water leaving the project area and entering adjacent dieback uninfested areas. | - Clean up area of actual or potential dieback infestation, if necessary. - Investigate and determine the cause or source of surface water movement. - Review practicality and relevance of dieback management procedures. - Review and improve drainage controls to further prevent surface water run-off occurring from the project area. - Engage a registered Phytophthora Dieback Interpreter to undertake sampling to determine whether dieback infestation has occurred, if necessary. | Project Manager |
| Water Use | Evidence of untreated water used within the project area. | - Immediately discontinue the use of untreated water. - Clean-up area of actual or potential dieback infestation, if necessary. - Review dieback management procedures to ensure water is treated prior to entry onto project area. - Improve procedures for treatment of project water use and provide awareness - for project employees. | Project Manager |

# Appendix E

Dieback Report