# ORD MRWA Barrier line - README

Updated process removes the need for the ORD to MX Transformation Spreadsheet - Blank

# Configuration

Copy:

MRWA-Overtaking_Sight_Visibility_Report-DesignSpeed.xsl

To:

C:\ProgramData\Bentley\OpenRoads Designer CE\Configuration\Organization-Civil\ANZ Design\Reports\Evaluation ANZ

# Process

- Run OpenRoads Sight Visibility

- Create report using MRWA-Overtaking_Sight_Visibility_Report-DesignSpeed.xsl

- Copy report data and past into ORD Barrier-Line-Spreadsheet Blank .xlsm

## Refer to video

ORD- MRWA_barrier_line_spreadsheet_EXAMPLE.mp4